PRODUCT REQUIREMENTS DOCUMENT

ROV ILLO

Integrated 8-Thruster Autonomous ROV System

| Document field | Value |
| --- | --- |
| Document type | Product Requirements Document (PRD) |
| Version | V0.3 — Engineering baseline for continued implementation |
| Status | Working engineering baseline; implementation-aligned; open items explicitly tracked |
| System focus | Mechanical + Electrical + Embedded + Control + GCS + Payload + Mission |
| Control architecture | Current baseline: simple state estimator → stabilized manual controller → generalized wrench → TAM/allocator → actuation; future LMPC after validated SysID model |
| Navigation baseline | HWT905 + pressure/depth; validated attitude/heading/depth only. No unsupported absolute or inertial-only XY navigation claim. |
| Tether | Blue Robotics zero-buoyancy tether; treated as an operational hydrodynamic disturbance, not a position sensor |
| Current implementation | tyrant-simple: Teensy safety authority, micro-ROS, stabilized MANUAL, HWT905, teleop/HIL/SysID infrastructure |
| Companion compute | Jetson Nano 4GB; ROS 2 Humble userspace planned in containerized companion workflow |
| Deferred hardware item | Real depth sensor installation/qualification remains pending; software/HIL path stays testable without it |

# 1. Document Purpose and Scope

Dokumen ini menjadi baseline requirement untuk membangun ROV ILLO sebagai satu sistem terintegrasi sampai hardware nyata, bukan hanya sebagai proof-of-concept software. PRD mendefinisikan apa yang harus dibangun, bagaimana subsystem berinteraksi, batas scope penelitian, dan bagaimana keberhasilan sistem akan diverifikasi.

## 1.1 Tujuan utama

Mendefinisikan kebutuhan sistem end-to-end untuk ROV 8-thruster.

Menjadi single source of truth antara mechanical, electrical, firmware, control, GCS, mission, payload, dan testing.

Memisahkan requirement produk/platform dari kontribusi ilmiah thesis.

Menyediakan acceptance criteria yang dapat diuji secara bertahap.

Mencegah scope creep dengan penandaan MUST / SHOULD / OPTIONAL / TBD.

## 1.2 Thesis boundary

ROV lengkap tetap dibangun sebagai experimental platform. Kontribusi ilmiah utama dibatasi pada pemodelan dinamika dan kendali kendaraan, khususnya integrasi model 6-DOF, hybrid CFD–grey-box System Identification, 6-DOF state estimation, 6-DOF LMPC, dan constrained thruster allocation. GCS, PCB, power, payload, dan mechanical system merupakan enabler/platform implementation kecuali secara eksplisit dipilih sebagai kontribusi tambahan.

## 1.3 V0.3 engineering baseline

V0.3 aligns the product requirements with the validated tyrant-simple development direction. The current firmware shall not claim XY position/velocity that is not supported by an appropriate aiding sensor. The present control baseline is stabilized MANUAL control and safety-first actuation; LMPC remains a future required research/control milestone after thruster characterization, SysID, and grey-box model validation.

# 2. Product Definition

ROV ILLO adalah underwater remotely operated vehicle bertether yang dirancang untuk operasi manual dan autonomous terbatas berbasis state estimation lokal/relatif. Vehicle memiliki 8 thruster, Teensy 4.1 sebagai real-time flight-control dan final safety authority, Jetson Nano 4GB sebagai companion gateway/mission/logger ROS 2, ESP32-S3 sebagai payload controller, serta custom GCS.

## 2.1 System concept

GCS
  │
Fathom-X / Zero-Buoyancy Tether
  │
Jetson Nano 4GB — gateway + mission supervisor + logger + ROS 2
  ├──────── Teensy 4.1 — real-time flight controller / safety authority
  │        ├─ HWT905 IMU + pressure/depth sensor
  │        ├─ Simple state estimator baseline (attitude / heading / depth)
  │        ├─ Stabilized MANUAL controller
  │        ├─ Future validated model-based / LMPC controller
  │        ├─ 6×8 TAM / constrained allocation
  │        └─ 8 thruster outputs
  └──────── ESP32-S3 — payload controller
           ├─ pH
           ├─ DO
           └─ Temperature

## 2.2 Core product goals

| ID | Goal | Priority |
| --- | --- | --- |
| G-001 | ROV dapat beroperasi underwater melalui tether. | MUST |
| G-002 | ROV mendukung MANUAL mode. | MUST |
| G-003 | ROV mendukung autonomous mission berbasis local/relative reference. | MUST |
| G-004 | ROV memiliki validated onboard state estimation for controller-required attitude, heading, angular-rate, and depth states; unsupported XY navigation shall not be claimed. | MUST |
| G-005 | ROV memiliki 6-DOF generalized-wrench control architecture with stabilized manual baseline and future model-based/LMPC capability after model validation. | MUST |
| G-006 | 8 thruster dialokasikan melalui TAM/QP. | MUST |
| G-007 | Payload pH/DO/temperature dapat diakuisisi. | MUST |
| G-008 | Custom GCS menyediakan monitoring, manual control, mission, dan diagnostics. | SHOULD |
| G-009 | Sistem memiliki logging dan fault handling. | MUST |

# 3. System Architecture

## 3.1 Node responsibilities

| Node | Primary role | Responsibilities | Real-time critical? |
| --- | --- | --- | --- |
| GCS | Operator interface | Dashboard, manual command, mission authoring, telemetry visualization, parameter UI, logs | No |
| Jetson Nano 4GB | Gateway / Mission Supervisor / Logger | Tether/GCS communication, ROS 2 integration, command routing, mission state machine, payload routing, high-level supervisory logic, logging, experiment support | No |
| Teensy 4.1 | Flight controller | Sensor acquisition, validated state estimation, stabilized real-time control, future LMPC, TAM/allocation, thruster commands, watchdog, safety authority, high-rate telemetry | Yes |
| ESP32-S3 | Payload controller | pH/DO/temp acquisition, sampling, payload health, payload packets | Moderate |
| PDB PCB | Power distribution | Battery input, protection, branch distribution, current/voltage monitoring as designed | Yes |
| Main Board PCB | Main electronics interface | Teensy/compute interfaces, sensor interfaces, ESC/control interfaces, communication and protection as designed | Yes |

## 3.2 Authority hierarchy

GCS → Jetson Nano → Teensy

Jetson Nano decides WHAT mission/reference should occur and provides high-level supervision/logging.
Teensy decides HOW the vehicle physically executes it and whether execution is safe.
ESP32-S3 manages payload sensing/sampling and reports payload data/health.

Mode transitions that affect propulsion shall be validated by the Teensy. A high-level request from GCS/Jetson shall not directly bypass Teensy safety checks.

# 4. Operating Modes and State Machine

| Mode | Description | Thruster behavior | Entry condition |
| --- | --- | --- | --- |
| BOOT | Hardware/firmware initialization | Disabled | Power-on/reset |
| IDLE | System ready, no vehicle motion command | Disabled / zero propulsion authority | Initialization passed |
| MANUAL | Operator commands motion | Enabled under safety authority | Valid communication + valid required state + accepted MANUAL request |
| AUTO | Mission/reference execution | Enabled through control stack | Mission accepted + controller/state valid |
| SAFE | Controlled safe state | Zero propulsion authority in current baseline; not automatic hold/surface | Heartbeat/command timeout, operator SAFE request, abort, or safety policy |
| FAULT | Critical error state | Safe output enforced | Critical fault latched |
| EMERGENCY | Emergency handling | As defined by safety design | Emergency request/condition |

# 5. Mechanical System Requirements

## 5.1 Frame and pressure housing

| ID | Requirement | Verification |
| --- | --- | --- |
| MECH-001 | Frame shall provide rigid mounting for 8 thrusters and pressure/equipment structures. | Inspection |
| MECH-002 | Mechanical design shall provide access for assembly, maintenance, and cable routing. | Inspection |
| MECH-003 | Pressure housing shall protect electronics to the validated operating depth. | Pressure test |
| MECH-004 | Sensor and payload mounts shall avoid unacceptable obstruction of thruster flow. | Inspection + water test |
| MECH-005 | Tether attachment shall be mechanically secure and strain-managed. | Inspection + pull/handling test |

## 5.2 Buoyancy and trim

CG and CB shall be estimated and, where practical, measured/validated.

Vehicle shall be trimmed for stable operating attitude.

Buoyancy material and mounting shall withstand expected depth/environment.

Trim shall consider battery, payload, tether, and connector mass/volume.

## 5.3 Hydrodynamic design

CAD/CFD geometry shall represent the as-built vehicle as closely as practical. Small unmodeled parts may remain; their effects will be addressed through real-vehicle testing and grey-box identification.

# 6. Propulsion System

## 6.1 8-thruster configuration

Baseline configuration: 8 thrusters, consisting of 4 vertical thrusters and 4 horizontal thrusters oriented approximately 45 degrees according to the approved mechanical geometry. Exact position vectors, force directions, sign conventions, and motor numbering are controlled by the mechanical/TAM ICD and shall not be assumed from a generic ROV.

| ID | Requirement | Verification |
| --- | --- | --- |
| PROP-001 | All 8 thruster channels shall be independently commandable. | Bench test |
| PROP-002 | Each thruster shall have a unique ID and defined positive force direction. | Inspection |
| PROP-003 | ESC command range and deadzone shall be characterized. | Thruster calibration |
| PROP-004 | Forward and reverse thrust response shall be measured where supported. | Thruster calibration |
| PROP-005 | TAM shall map generalized wrench to 8 actuator commands. | Simulation + bench |
| PROP-006 | TAM shall enforce individual actuator limits. | Simulation + fault test |
| PROP-007 | System shall detect unavailable/faulted thruster channels at least at command/health level. | Fault injection |

# 7. Electrical and Power System

Electrical design is already substantially implemented as two PCBs: PDB and Main Board. The PRD therefore treats PCB design as an existing subsystem that must be brought up and verified, while final numerical ratings remain controlled by the electrical design package.

## 7.1 PDB

| ID | Requirement | Status |
| --- | --- | --- |
| PWR-001 | Distribute battery power to propulsion and electronics branches as designed. | MUST |
| PWR-002 | Provide branch protection appropriate to battery and load ratings. | MUST |
| PWR-003 | Provide voltage/current monitoring where defined by PCB design. | SHOULD |
| PWR-004 | Prevent unsafe reverse/short conditions within defined electrical architecture. | MUST |
| PWR-005 | Power budget shall account for 8 thruster peak/continuous load and compute loads. | MUST |

## 7.2 Main Board

Host/interface the Teensy and required sensor/control/communication interfaces.

Provide deterministic, labeled connectors and test points consistent with schematic.

Respect 3.3 V logic constraints of Teensy/ESP32 interfaces.

Provide serviceability for bench testing before enclosure closure.

## 7.3 Power budget

| Parameter | Target/Value | Status |
| --- | --- | --- |
| Battery voltage | TBD from final battery configuration | TBD |
| Battery capacity | TBD | TBD |
| Peak propulsion current | TBD | TBD |
| Continuous propulsion current | TBD | TBD |
| Logic rail(s) | TBD from PCB design | TBD |
| Expected endurance | TBD | TBD |
| Low-voltage threshold | TBD | TBD |

# 8. Embedded Computing and Firmware

## 8.1 Teensy firmware architecture

Teensy Firmware
├─ HAL / Drivers
├─ Communication / micro-ROS
├─ Sensor Manager
├─ Simple State Estimator baseline
│  ├─ Roll/pitch complementary filter
│  ├─ Yaw + gyro-bias 2-state KF
│  └─ Pressure → positive-down depth estimator
├─ Safety / Vehicle Mode / Watchdogs
├─ Stabilized MANUAL Controller
├─ Reference / Mission / Emergency interfaces
├─ SysID runtime
├─ TAM / Allocator
├─ Actuation backend (DISABLED / MOCK / PWM)
└─ Future validated model-based / LMPC controller

## 8.2 Jetson Nano software architecture

Jetson Nano
├─ ROS 2 Humble companion runtime
├─ micro-ROS Agent / serial integration
├─ Tether / GCS Communication
├─ Mission Manager
├─ Reference / Command Router
├─ Payload Router
├─ Supervisor / Health Monitor
├─ Data Logger / Experiment Logger
└─ GCS / camera / diagnostics integration as deployed

## 8.3 ESP32-S3 software architecture

ESP32-S3
├─ I²C Sensor Drivers
├─ Payload Acquisition
├─ Sampling State Machine
├─ Calibration/Configuration
├─ Health Monitor
└─ Communication / Data Packetizer

# 9. Sensor System

| Sensor | Role | Interface | Primary node | Status |
| --- | --- | --- | --- | --- |
| HWT905 9-axis IMU | Angular rate, acceleration, attitude-related measurements | UART serial (production target) | Teensy | Real sensor bench path validated; final chassis mounting/sign qualification still required |
| Blue Robotics-compatible pressure/depth sensor (FullDepth/MS5837 path) | Pressure/depth/temperature | I²C | Teensy | Software + smoke-test path implemented; physical installation/qualification currently deferred |
| pH sensor (Atlas Scientific EZO) | Water quality | I²C / defined carrier interface | ESP32-S3 | Planned |
| DO sensor (Atlas Scientific EZO) | Dissolved oxygen | I²C / defined carrier interface | ESP32-S3 | Planned |
| Temperature | Payload/environment | I²C/analog as selected | ESP32-S3 | Planned |

## 9.1 Sensor health

Timeout detection

Range/validity checks

Initialization status

Data freshness

Calibration state

Fault flag propagation to GCS

# 10. Navigation and State Estimation

## 10.1 Navigation baseline

ROV does not have GPS underwater and the baseline does not include DVL or USBL. V0.3 therefore forbids claiming accurate absolute/local XY position from IMU integration alone. The validated baseline estimator is limited to attitude/heading/angular-rate/depth states and explicit validity flags. Any future XY position/velocity estimate must identify its source and be separately validated.

## 10.2 Validated baseline state definition

x_simple = [roll, pitch, yaw, p, q, r, yaw_gyro_bias, depth] with attitude_valid, heading_valid, and depth_valid flags.

The product architecture may later expand the estimator if validated aiding sensors and control requirements justify it. Expanded states shall not silently replace the reduced baseline or imply navigation accuracy that has not been verified.

## 10.3 State-estimation requirements

| ID | Requirement | Verification |
| --- | --- | --- |
| NAV-001 | Estimator shall provide bounded roll, pitch, yaw, body angular rates, yaw-bias estimate, and depth when corresponding sensors are valid. | SIL + hardware |
| NAV-002 | Estimator shall use HWT905 IMU data and pressure-derived depth without requiring depth availability for attitude-only MANUAL operation. | Simulation + hardware |
| NAV-003 | Estimator shall expose explicit attitude_valid, heading_valid, depth_valid, and health/status information. | Fault test |
| NAV-004 | Estimator shall reject stale/invalid/non-finite sensor data and fail closed at controller gates when a required state is unavailable. | Fault test |
| NAV-005 | Navigation software shall not claim absolute or accurate local XY position without an appropriate validated aiding source. | Inspection + test |
| NAV-006 | Any future expanded estimator shall be validated in SIL/HIL and against hardware logs before being used as control authority. | SIL/HIL + review |

# 11. Dynamic Model, CFD and System Identification

## 11.1 Baseline dynamic model

M ν_dot + C(ν)ν + D(ν)ν + g(η) = τ

The model shall distinguish rigid-body properties, added mass, hydrodynamic damping/drag, and restoring forces. The product does not require a perfect global model; the controller model must be sufficiently accurate over the intended operating envelope.

## 11.2 CFD role

Estimate hydrodynamic forces/moments and initial drag coefficients.

Provide physics-based parameter priors/initial values.

Support sensitivity studies and geometric design decisions.

Not be treated as automatic ground truth for the as-built tethered vehicle.

## 11.3 Grey-box SysID

CFD / CAD priors
      ↓
Initial dynamic model
      ↓
Real ROV excitation tests (with actual tether)
      ↓
Grey-box parameter estimation
      ↓
Hybrid calibrated model
      ↓
Independent validation dataset

## 11.4 Planned identification experiments

| Experiment | Objective | Primary motions/signals |
| --- | --- | --- |
| EXP-01 Thruster characterization | PWM/command → thrust response | T1…T8, command, current if available |
| EXP-02 Surge | Estimate X-direction dynamics | u, X, thruster commands |
| EXP-03 Sway | Estimate Y-direction dynamics | v, Y, thruster commands |
| EXP-04 Heave | Estimate Z-direction dynamics | w, Z, depth, thruster commands |
| EXP-05 Yaw | Estimate yaw dynamics | r, N, thruster commands |
| EXP-06 Roll | Estimate roll dynamics | p, K, attitude, thruster commands |
| EXP-07 Pitch | Estimate pitch dynamics | q, M, attitude, thruster commands |
| EXP-08 Dynamic excitation | Improve added-mass/dynamic parameter estimates | acceleration/deceleration datasets |
| EXP-09 Validation | Test generalization on unseen inputs | all selected signals |

Identification shall avoid estimating every coefficient simultaneously unless data supports parameter identifiability. Rigid-body quantities should come from measured/CAD properties wherever practical, while hydrodynamic terms are estimated from experiments and/or CFD-informed priors.

# 12. 6-DOF Control Architecture

## 12.1 Control objective

The control architecture targets generalized BODY-FRD wrench τ = [X Y Z K M N]^T. The current operational baseline uses direct surge/sway effort, depth-rate/depth-hold for heave when depth is valid, heading-rate/heading-hold for yaw, and stabilized roll/pitch references. Roll/pitch stabilization remains active even when operator roll/pitch commands are zero.

## 12.2 Current stabilized MANUAL baseline

MANUAL control shall require fresh manual command, propulsion permission, valid control timing, valid attitude state, and non-fault estimator state. Missing required state shall produce zero wrench. Command freshness timeout is a safety gate; host-heartbeat loss shall force SAFE according to Teensy safety policy. Heave/depth functions that require depth shall remain unavailable while depth_valid is false.

## 12.3 Future LMPC architecture

Reference / Mission
       ↓
6-DOF State Reference
       ↓
Validated controller state / future expanded estimator
       ↓
6-DOF Linearized Model
       ↓
LMPC
       ↓
Desired wrench τd = [X Y Z K M N]^T
       ↓
TAM / QP

## 12.4 Future LMPC requirements

| ID | Requirement | Priority |
| --- | --- | --- |
| CTRL-001 | LMPC shall operate on a defined 6-DOF prediction model. | MUST |
| CTRL-002 | LMPC shall use validated controller state inputs appropriate to the identified model; any expanded estimator used for LMPC shall be separately validated. | MUST |
| CTRL-003 | LMPC shall enforce relevant state/input constraints. | MUST |
| CTRL-004 | LMPC shall expose solver health/failure status. | MUST |
| CTRL-005 | LMPC weighting and horizons shall be configurable. | SHOULD |
| CTRL-006 | Closed-loop performance shall be validated in simulation before hardware. | MUST |

# 13. Thruster Allocation / TAM / QP

## 13.1 Mathematical definition

τ = B T
B ∈ R^(6×8)
τ = [X Y Z K M N]^T
T = [T1 T2 … T8]^T

Each thruster contribution shall be computed from its actual force direction d_i and position vector r_i relative to the chosen body reference point. The allocation matrix shall be generated from approved as-built geometry, not copied from a generic BlueROV layout. Production actuation shall remain fail-closed until thruster numbering, geometry, direction signs, and actuator limits are frozen and verified.

## 13.2 QP objective

min_T  || B T - τd ||_Q^2 + λ ||T||_R^2
subject to T_min ≤ T ≤ T_max

## 13.3 Required tests

Single-axis wrench reconstruction: Surge, Sway, Heave, Roll, Pitch, Yaw.

Combined 6-DOF wrench reconstruction.

Actuator saturation scenarios.

At least one simulated thruster failure/reduced-authority scenario.

Sign-convention and coordinate-frame verification.

# 14. Mission and Autonomous Operation

## 14.1 Mission philosophy

Because GPS is unavailable underwater and no DVL/USBL is planned, mission planning shall not require absolute geographic waypoints as a baseline. Mission commands shall be expressed as depth, heading, velocity, time, actions, and optionally short-duration relative motion targets.

## 14.2 Mission primitives

| Primitive | Example | Requires absolute position? |
| --- | --- | --- |
| DIVE | Depth = 5 m | No |
| HOLD_DEPTH | 5 m for 10 s | No |
| SURGE_VELOCITY | u = 0.3 m/s for 10 s | No |
| SWAY_VELOCITY | v = 0.2 m/s for 5 s | No |
| HEAVE_RATE | w reference | No |
| SET_HEADING | ψ = 90° | No |
| ROTATE | Yaw +90° | No |
| HOLD_ATTITUDE | φ=0°, θ=0° | No |
| SAMPLE | Take water sample | No |
| RELATIVE_MOVE | Δx/Δy over short interval | Relative/local estimate |
| ABORT | Switch to safe behavior | No |

## 14.3 Mission state machine

MISSION START
  ↓
DIVE
  ↓
HOLD DEPTH
  ↓
MOVE / ROTATE
  ↓
HOLD / SAMPLE
  ↓
NEXT ACTION
  ↓
MISSION COMPLETE

At any step: timeout / fault / operator abort → SAFE / ABORT

## 14.4 Return behavior

Exact geographic return-to-home is not a baseline requirement without an external position reference. The system may support approximate return-by-replay or relative return sequences, but the GCS and mission specification shall label this as approximate and shall not claim absolute positioning accuracy.

# 15. Custom Ground Control Station (GCS)

GCS akan dibangun secara custom dengan filosofi yang mengambil pola interaksi dari QGroundControl/BlueOS, tetapi tidak menjadi clone. Fokusnya adalah kebutuhan ROV ILLO: monitoring vehicle, manual control, mission, payload, diagnostics, parameters, dan logs.

## 15.1 GCS modules

| Module | Minimum capability | Priority |
| --- | --- | --- |
| Dashboard | Depth, attitude, state, battery, connection, mode, health | MUST |
| Manual Control | Joystick/manual motion command, enable/disable, mode request | MUST |
| Mission Planner | Sequence editor for depth/heading/velocity/action missions | MUST |
| Payload | pH/DO/temperature, sampling state | SHOULD |
| Diagnostics | Node/sensor/controller/thruster health | MUST |
| Parameters | Read/write approved configuration set | SHOULD |
| Logs | Browse/export mission and telemetry logs | SHOULD |
| System Setup | Calibration/status/configuration helpers | SHOULD |

## 15.2 GCS navigation display

Primary display shall emphasize vehicle state and mission state rather than assuming a GPS map. A local mission canvas may visualize relative trajectory, reference commands, and estimated vehicle track; it must clearly label estimated/local coordinates.

## 15.3 Manual control policy

Operator may command exposed motion axes, but Teensy retains final authority for safety and output limits. Roll and pitch default to stabilization/assist behavior. Manual command publication requires a deadman guard. Mode request buttons request state transitions; they do not directly grant propulsion authority.

## 15.4 PS controller baseline mapping

The following mapping is the current validated Bluetooth controller baseline. Button/axis indices come from tyrant_joy_probe and shall remain configurable in ps_controller.yaml. Mode requests are edge-triggered and must still be accepted by Teensy safety logic.

| Physical input | ROS Joy index | Function | Safety / notes |
| --- | --- | --- | --- |
| Cross | button[0] | Request MANUAL | Requested mode = 2; Teensy may reject if safety prerequisites are not met. |
| Square | button[2] | Request IDLE | Requested mode = 1; normal zero-propulsion state. |
| Circle | button[1] | Request SAFE | Requested mode = 4; current SAFE means zero propulsion, not active surface/hold. |
| Triangle | button[3] | Set depth surface reference | Only meaningful when pressure/depth sensor path is available and policy allows request. |
| L1 | button[4] | Deadman / manual enable guard | Must be held for active ManualCommand publication. |
| R1 | button[5] | Reserved | No safety-critical function until explicitly assigned and tested. |
| Left stick X | axis[0] | Sway | Sign/inversion verified through /tyrant/manual/command and BODY-FRD convention. |
| Left stick Y | axis[1] | Surge | Forward sign shall map to positive surge after configured inversion/deadzone. |
| Right stick X | axis[3] | Yaw | Heading-rate/heading-hold input path. |
| Right stick Y | axis[4] | Reserved / future use | No production function until intentionally assigned and verified. |

# 16. Communication and Protocol

## 16.1 Network topology

GCS <-> Fathom-X <-> Tether <-> Jetson Nano
Jetson Nano <-> Teensy 4.1 (micro-ROS serial)
Jetson Nano <-> ESP32-S3 / payload path as implemented

The tether is the communication path, not a navigation sensor. Its zero-buoyancy characteristic shall be preserved in the mechanical/operational description; hydrodynamic drag and tether geometry remain test variables.

## 16.2 Packet structure

HEADER | VERSION | SOURCE | DESTINATION | MSG_ID | SEQUENCE | TIMESTAMP | LENGTH | PAYLOAD | CRC

## 16.3 Message classes

| Message | Purpose | Direction |
| --- | --- | --- |
| HEARTBEAT | Node link/health | Bidirectional |
| MODE | Mode requests/status | GCS/Jetson ↔ Teensy |
| REFERENCE | Motion/control reference | Jetson/GCS → Teensy |
| COMMAND | Manual command | GCS/Jetson → Teensy |
| STATE | Estimated vehicle state | Teensy → Jetson/GCS |
| SENSOR | Sensor data/health | Node → Jetson/Teensy |
| THRUSTER | Command/status | Teensy ↔ Jetson/GCS |
| MISSION | Mission definition/status | GCS ↔ Jetson |
| PAYLOAD | pH/DO/temp | ESP32 → Jetson/GCS |
| PARAMETER | Configuration | Jetson/GCS ↔ Teensy/ESP32 |
| FAULT | Fault/event | Node → Jetson/GCS |

# 17. Payload System

| Requirement ID | Requirement | Verification |
| --- | --- | --- |
| PAY-001 | ESP32-S3 shall acquire pH, DO, and temperature measurements. | Bench test |
| PAY-002 | Payload readings shall include timestamps and validity/health status. | Protocol test |
| PAY-003 | Sampling shall be controlled by a deterministic sampling state machine. | Functional test |
| PAY-004 | Payload data shall be forwarded to Jetson Nano/GCS. | Integration test |
| PAY-005 | Calibration/configuration state shall be trackable. | Inspection + test |

# 18. Data Logging and Diagnostics

## 18.1 Teensy high-rate log

Raw/filtered IMU

Depth

Simple state + validity / future controller state

Reference

LMPC state/output

TAM output

Individual thruster commands

Timestamps/sequence numbers

Faults and mode transitions

## 18.2 Jetson Nano / companion log

GCS commands

Mission definition/progress

Telemetry

Payload data

Mode transitions

Communication diagnostics

System-level events

## 18.3 Log quality

| Requirement | Target |
| --- | --- |
| Timestamp source | Consistent monotonic time base within each node; cross-node synchronization strategy TBD |
| Data integrity | Sequence numbers + packet validity checks |
| Export | CSV/JSON or project-defined machine-readable format |
| Fault traceability | Events shall include timestamp and subsystem/source |

# 19. Safety, Failsafe and Fault Management

| Condition | Detection | Expected action |
| --- | --- | --- |
| Communication / command timeout | Host heartbeat watchdog, manual-command freshness, micro-ROS link state | Zero wrench and/or force transition to SAFE according to Teensy safety policy |
| Invalid required sensor/state | Validity/range/timeout checks | Block affected controller path; do not synthesize missing state authority |
| Estimator invalid | Estimator/state health flag | Block/exit AUTO as configured |
| LMPC solver failure | Solver status | Fallback/SAFE behavior |
| QP infeasible | QP status | Fallback allocation/SAFE behavior |
| Thruster fault | Health/current/command diagnostic as available | Reconfigure/disable channel or SAFE |
| Low battery | Voltage/current monitor | Warn and abort/return strategy |
| Overcurrent/thermal | Protection/telemetry | Disable affected branch or SAFE |
| Leak/pressure fault | Leak/pressure sensor if implemented | Emergency response TBD |
| Joystick / teleop stale | Joy freshness + teleop timeout | Neutral/stop ManualCommand; request SAFE after configured timeout if enabled |

Exact fail-safe outputs must be validated on the bench before water testing. Safety behavior must never rely solely on Jetson application logic. In the current baseline SAFE means zero propulsion authority; it is intentionally distinct from active depth hold, automatic surfacing, or return behavior.

# 20. Simulation and Development Verification

## 20.1 SIL

Development SIL/HIL path
      ↓
Reduced / validated simulation state
      ↓
Mission / Manual / SysID command source
      ↓
Real Teensy controller + safety
      ↓
Generalized wrench
      ↓
Synthetic or validated TAM / allocator
      ↓
Mock thruster commands / plant feedback

Full hydrodynamic 6-DOF + LMPC validation is a later stage after the identified model is available.

## 20.2 HIL / bench

Real Teensy executing production controller/safety code with simulated state input (teensy41-hil) and no physical PWM.

Jetson/laptop ROS 2 companion stack running micro-ROS agent, teleop, simulator, logger, and supervisory nodes as applicable.

Real ESP32 packet flow with simulated payload values.

ESC/thruster bench test before full-water operation.

# 21. Verification and Validation Plan

| Test ID | Area | Pass criterion (baseline) |
| --- | --- | --- |
| VVT-001 | PCB power-on | All intended rails within design limits; no abnormal heating |
| VVT-002 | Teensy bring-up | Firmware boots and health packet is valid |
| VVT-003 | Jetson↔Teensy comm | Bidirectional ROS/micro-ROS traffic, heartbeat, timeout, reconnect, and boot-order behavior pass |
| VVT-004 | Jetson↔ESP32 comm | Payload packets received reliably |
| VVT-005 | Individual thruster | Each channel responds correctly with correct sign |
| VVT-006 | TAM | Reconstructed wrench matches requested wrench within defined tolerance |
| VVT-007 | State estimator | Required attitude/heading/depth states remain bounded, validity flags behave correctly, and unsupported XY is not claimed |
| VVT-008 | LMPC | Future LMPC tracking meets defined target in SIL before hardware closed-loop |
| VVT-009 | Mission | Mission sequence completes without unsafe mode transitions |
| VVT-010 | Failsafe | Heartbeat, command, controller-disconnect, sensor-invalid, and fault cases produce defined zero/SAFE behavior |
| VVT-011 | Payload | pH/DO/temp values acquired and logged |
| VVT-012 | Water integration | Vehicle remains mechanically/electrically safe and stable |
| VVT-013 | Model validation | Hybrid model predicts validation dataset better or comparably within defined metric |
| VVT-014 | End-to-end | Representative mission completes with traceable logs |
| VVT-015 | Controller / teleop | Bluetooth/USB Joy → teleop → ManualCommand → Teensy works; Cross/MANUAL, Square/IDLE, Circle/SAFE and L1 deadman behave as specified |
| VVT-016 | micro-ROS reconnect | Agent restart/disconnect recovers without Teensy reset and without unsafe propulsion |
| VVT-017 | HIL baseline | Real Teensy + simulated state + mock actuation closes the software loop without physical thruster PWM |

# 22. Development / Build Phases

Build phase diubah dari urutan serial menjadi parallel development karena mechanical dan electrical sudah berada pada tahap matang/menunggu komponen. Tujuan fase sekarang adalah membuat software/control stack siap sebelum PCB selesai dirakit.

| Phase | Work package | Status / priority |
| --- | --- | --- |
| P0 | Mechanical design / print preparation | Substantially complete |
| P1 | PDB + Main Board fabrication / component procurement | In progress / waiting components |
| P2 | Local ROS 2 + micro-ROS + Teensy + real HWT905 + teleop baseline | PASS baseline; maintain regression |
| P3 | Jetson Nano companion deployment + logging + persistent device setup | NEXT major deployment step |
| P4 | Hardware assembly / PCB bring-up / final sensor bus qualification | After hardware availability |
| P5 | Thruster/ESC characterization + numbering/sign/geometry freeze | Before production TAM/QP |
| P6 | Real 6×8 TAM + constrained allocator verification | After P5 |
| P7 | HIL/bench integrated safety and control validation | Parallel after P3/P6 as applicable |
| P8 | Grey-box SysID campaign with actual tether | After safe bench/water-ready bring-up |
| P9 | Validated 6-DOF dynamic model freeze | After SysID train/validation split |
| P10 | LMPC synthesis/tuning + SIL validation | After P9; do not freeze earlier |
| P11 | Custom GCS / mission / experiment tooling | Parallel, interface-driven |
| P12 | Controlled water testing + model/controller validation | After bench safety gate |
| P13 | Final thesis/product verification and traceable evidence | Final stage |

# 23. Current Execution Plan — What to Build While Waiting for Components

| Priority | Deliverable | Definition of done |
| --- | --- | --- |
| 1 | Maintain local known-good baseline | Regression tests, real HWT905, micro-ROS reconnect, MANUAL/SAFE/IDLE teleop and watchdog behavior stay green |
| 2 | Jetson Nano runtime | Agent, ROS 2 packages, persistent serial device, logging/diagnostics, headless startup validated |
| 3 | Hardware/sensor integration | PCB rails/interfaces, final HWT mounting, real depth sensor installation and qualification |
| 4 | Thruster characterization | Forward/reverse response, deadzone, limits, numbering and force signs measured |
| 5 | Real TAM / allocator | Matrix generated from as-built geometry; single-axis + combined wrench + saturation tests pass |
| 6 | HIL / bench control gate | Real Teensy safety/controller + simulated state/mock actuation passes timeout/fault/boot-order tests |
| 7 | SysID protocol + datasets | Excitation profiles, synchronized logs, experiment IDs, train/validation data produced |
| 8 | Grey-box model validation | CAD/CFD priors + identified parameters validated on unseen data |
| 9 | LMPC | Controller designed only from validated model; constraints/fallback/solver health tested in SIL |
| 10 | GCS / mission integration | Manual, modes, diagnostics, logs, experiment and mission workflow integrated without bypassing Teensy authority |

# 24. Custom GCS Functional Requirements

| ID | Requirement |
| --- | --- |
| GCS-001 | Display current mode and system health prominently. |
| GCS-002 | Display depth and 6-DOF attitude/state. |
| GCS-003 | Allow joystick/manual motion command generation with explicit deadman enable guard and IDLE/MANUAL/SAFE mode requests. |
| GCS-004 | Allow creation/editing of motion-based autonomous mission sequences. |
| GCS-005 | Display mission progress, active action, and abort control. |
| GCS-006 | Display payload measurements and sampling status. |
| GCS-007 | Display sensor, node, controller, and thruster diagnostics. |
| GCS-008 | Display local/relative estimated track with explicit accuracy/source labeling. |
| GCS-009 | Record/export logs and mission events. |
| GCS-010 | Support controlled parameter access and versioning. |
| GCS-011 | Never bypass Teensy safety authority to command propulsion or mode transitions. |
| GCS-012 | Support simulated vehicle mode for development before hardware integration. |
| GCS-013 | Display or log controller/teleop freshness and make loss of operator input diagnosable. |

# 25. Interface Control Document (ICD) — Required Artifacts

| Artifact | Owner / source | Status |
| --- | --- | --- |
| Mechanical geometry + thruster coordinates | Mechanical CAD | Required |
| Thruster numbering/sign convention | Mechanical + Control | Required |
| PDB schematic/pinout | Electrical | Available/in development |
| Main Board schematic/pinout | Electrical | Available/in development |
| Sensor bus/pin map | Electrical/Firmware | Required |
| Communication / ROS interface definition | Software | Implemented baseline; version-control with firmware interfaces |
| Validated state interface definition | Control / Estimation | SimpleState baseline implemented; expanded state TBD only if justified |
| TAM matrix generation script | Control | Pending real geometry freeze |
| Parameter/config schema | Software / Control | Partially implemented; continue versioning |
| Fault / reason code list | System | Partially implemented; consolidate before GCS freeze |
| Mission command schema | GCS/Jetson | To build |
| Controller mapping / teleop safety policy | Software / HMI | Baseline defined in ps_controller.yaml + PRD V0.3; keep configurable |

# 26. Key Performance Requirements — TBD Register

Numerical targets are intentionally not invented in this draft. They shall be filled from the actual mechanical design, battery, thruster, test environment, and thesis objectives.

| Parameter | Required value | Owner / basis | Status |
| --- | --- | --- | --- |
| Maximum operating depth | TBD | Pressure housing rating + test | Open |
| Maximum surge speed | TBD | Thruster/CAD/water test | Open |
| Maximum sway speed | TBD | Thruster/TAM | Open |
| Maximum heave speed | TBD | Thruster/TAM | Open |
| Maximum angular rates | TBD | Vehicle/control envelope | Open |
| Depth tracking error | TBD | Control objective | Open |
| Heading tracking error | TBD | Control objective | Open |
| Roll stabilization error | TBD | Control objective | Open |
| Pitch stabilization error | TBD | Control objective | Open |
| LMPC cycle rate | TBD | Teensy compute benchmark | Open |
| Estimator cycle rate | TBD | Sensor rate + Teensy compute benchmark | Open |
| Telemetry latency | TBD | Network test | Open |
| Battery endurance | TBD | Power budget + mission profile | Open |
| Payload sampling rate | TBD | Sensor requirement | Open |

# 27. Requirement Traceability Strategy

Setiap requirement system akan ditelusurkan melalui design → implementation → verification result. Requirement ID tidak boleh hilang ketika desain berubah.

REQUIREMENT
   ↓
SUBSYSTEM DESIGN
   ↓
IMPLEMENTATION
   ↓
TEST PROCEDURE
   ↓
TEST RESULT
   ↓
PASS / FAIL / WAIVED

# 28. Scope Boundary: Must / Should / Optional

| Category | MUST for baseline product | SHOULD | OPTIONAL / defer |
| --- | --- | --- | --- |
| Vehicle | 8 thrusters, mechanical frame, pressure housing, PDB + Main Board, safe power | Serviceability improvements | Major mechanical redesign |
| Embedded | Teensy real-time control/safety, Jetson gateway/mission/logger, ESP32 payload | Parameter infrastructure | Distributed redundancy |
| Navigation | HWT905 + depth, validated attitude/heading/depth state, explicit validity; no unsupported XY claim | Expanded relative estimation only when supported by validated aiding/model evidence | DVL/USBL |
| Control | Stabilized MANUAL baseline, generalized 6-DOF wrench architecture, real 6×8 TAM/QP, future LMPC after validated model | Fault-aware allocation, assisted references, controller diagnostics | Adaptive/online MPC |
| Modeling | 6-DOF model, CFD priors, grey-box SysID, validation | Coupled parameter studies | Full CFD of every accessory |
| Mission | Depth/heading/velocity/action mission | Relative track visualization | Global georeferenced mission |
| GCS | Dashboard, controller/teleop, modes, manual, mission, diagnostics | Payload/parameter/log modules | Complex map/terrain rendering |
| Safety | Watchdog, timeout, fault handling | Redundant safety paths | Advanced fault prediction |

# 29. Open Issues / Items Requiring Confirmation

Exact thruster model, positions, orientations, numbering, and force directions for the 6×8 TAM.

Final battery voltage/capacity and electrical protection ratings.

Exact PDB/Main Board interfaces and pin assignments from the latest schematic.

Final maximum depth and pressure-housing validation target.

Final HWT905 production UART, output rates, chassis mounting transform, and sign qualification.

Final pressure/depth sensor hardware installation, I²C address/interface, surface-reference operating procedure, and qualification.

Final mission definition: whether local relative displacement will use integrated velocity, timed velocity, or another short-horizon method.

LMPC sampling time, horizons, weights, constraints, and fallback strategy shall remain open until the grey-box model and compute benchmark are validated.

Final heave trigger mapping and any additional controller-button assignments beyond the validated V0.3 baseline.

Final payload sampling protocol and the mentioned EAS/DSRA procedure if it is part of the product requirement.

Final GCS technology stack and deployment packaging; Jetson Nano is already fixed as companion compute.

Exact performance tolerances for depth, attitude, tracking, latency, endurance, and model validation.

Real thruster geometry, ESC calibration curves, deadzone, and direction signs must be frozen before production allocator enable.

Depth hardware is temporarily deferred; HIL must cover depth-dependent software paths until the physical sensor can be installed.

AUTO/LMPC shall remain gated until required state/model/controller readiness is explicitly validated.

# 30. Review Checklist for V0.3

Gunakan checklist ini saat Anda mengoreksi PRD. Fokus utama adalah menemukan requirement yang belum tercantum, asumsi yang salah, atau bagian yang sebenarnya tidak akan dibangun.

| Review item | Check |
| --- | --- |
| Mechanical geometry and thruster layout match latest CAD | □ |
| PDB and Main Board responsibilities match latest schematics | □ |
| All sensors that will actually be purchased/used are listed | □ |
| Validated baseline state definition and validity policy are correct | □ |
| LMPC is correctly gated after SysID/model validation | □ |
| 6×8 TAM geometry/sign conventions are correctly represented | □ |
| Mission does not depend on GPS/DVL/USBL | □ |
| Zero-buoyancy tether is represented correctly as tether, not navigation sensor | □ |
| Custom GCS modules match intended product | □ |
| Power/failsafe assumptions are correct | □ |
| Payload requirements are complete | □ |
| CFD + SysID methodology matches thesis plan | □ |
| Testing sequence is realistic | □ |
| No critical subsystem has been omitted | □ |
| Numbers marked TBD that should already be fixed are identified | □ |
| Jetson Nano is consistently used as companion compute throughout the PRD | □ |
| Controller mapping Cross=MANUAL, Square=IDLE, Circle=SAFE, Triangle=surface, L1=deadman matches tested teleop config | □ |
| SAFE is documented as zero propulsion, not active hold/surface | □ |
| No section claims unsupported IMU-only XY navigation | □ |
| Production TAM/actuation remains fail-closed until real geometry/calibration is frozen | □ |

# 31. Baseline End-to-End Architecture

┌────────────────────────────┐
│         CUSTOM GCS         │
│ Manual | Mission | Logs    │
└─────────────┬──────────────┘
              │ Ethernet / Fathom-X tether
              ▼
┌────────────────────────────┐
│      JETSON NANO 4GB       │
│ Gateway | Mission | Logger │
│ ROS 2 | Teleop | HIL tools │
└─────────────┬──────────────┘
              │ micro-ROS serial
              ▼
┌────────────────────────────┐
│        TEENSY 4.1          │
│ Sensors | State | Safety   │
│ Manual Ctrl | TAM | SysID  │
└──────┬──────────────┬──────┘
       │              │
 HWT905 + Depth   8 Thrusters
       │
       └────► validated attitude/heading/depth baseline

ESP32-S3 remains the payload controller for pH/DO/temperature.
Navigation policy: no unsupported absolute/local XY claim without a validated aiding source.
Control roadmap: stabilized MANUAL → thruster characterization/TAM → SysID/validated model → LMPC.

# 32. Versioning and Change Control

V0.3 is the implementation-aligned engineering baseline. It supersedes V0.2 for forward development by adopting Jetson Nano, the tyrant-simple validated state/control baseline, explicit controller mapping and safety semantics, and the staged SysID→validated-model→LMPC roadmap. Changes that affect interfaces, state definitions, thruster geometry, PCB pinout, communication protocol, safety semantics, or thesis contribution shall continue to trigger controlled version updates.

## 32.1 V0.3 change record

V0.3 adopts Jetson Nano 4GB as companion compute; aligns the current estimator/control baseline to validated attitude, heading and depth states; documents stabilized MANUAL fail-closed gates and PS controller mapping (Cross->MANUAL, Square->IDLE, Circle->SAFE, Triangle->surface reference, L1->deadman); marks real depth hardware as temporarily deferred with HIL coverage; and fixes the forward roadmap as local baseline -> Jetson -> hardware/bench -> thruster characterization/TAM -> SysID/model validation -> LMPC -> water validation.

| Version | Meaning |
| --- | --- |
| V0.x | Design under review / requirements evolving |
| V1.0 | Architecture and core requirements frozen for implementation |
| V1.x | Controlled implementation changes after freeze |
| V2.x | Major architecture change or substantial new capability |