Tyrant Simple — Local Bring-up, Build, Test & Troubleshooting Guide

Laptop baseline before Jetson deployment

v1.3 — 2026-10-07

Purpose: establish a known-good local ROS 2 + PlatformIO + Teensy validation baseline before Jetson deployment.

# Contents

1–3 Scope, architecture, workspace structure

4–9 Local ROS/PlatformIO environment setup

10–14 Firmware profiles, tests, sensors, micro-ROS reconnect

15–21 Timing, nodes, topics, configs, mode/safety

22–27 Functional validation: sensors, Bluetooth teleop/controller mapping, mock, HIL, SysID, mission/emergency

28 Troubleshooting matrix

29–30 Code map

31–34 Daily workflow, Jetson gate, recovery commands

35 Validation notes and limitations

# 1. Tujuan dan scope

Dokumen ini adalah prosedur bring-up lokal untuk repository tyrant-simple sebelum workflow dipindahkan ke Jetson. Targetnya adalah membuat laptop menjadi known-good reference environment: ROS 2 workspace dapat dibuild, Python tests lulus, semua firmware profile dapat dikompilasi, Teensy regression tests dapat dijalankan, micro-ROS dapat connect/reconnect tanpa reset Teensy, sensor dapat divalidasi, teleop/HIL/SysID dapat diuji, dan error umum dapat didiagnosis dengan urutan yang konsisten.

Dokumen ini tidak menganggap model HIL sebagai model kendaraan final dan tidak menganggap PWM/geometry sintetis sebagai konfigurasi hardware production. Build production tetap fail-closed sampai geometri thruster dan karakterisasi actuator nyata tersedia.

Revisi v1.3 menyelaraskan mapping tombol dengan PRD V0.3, memperjelas gate Jetson saat sensor depth fisik ditunda, dan memperbaiki diagnosis freshness command. Status PASS pada bagian 23 berasal dari catatan sesi v1.2 tanggal 2026-10-07; revisi ini tidak menyatakan pengujian hardware telah diulang.

# 2. Baseline arsitektur yang sedang diuji

Laptop ROS 2 / GCS

│

├─ joy_node → tyrant_teleop → /tyrant/manual/command

├─ tyrant_sim (HIL) ↔ Teensy mock-thruster stack

├─ SysID excitation / rosbag

└─ micro_ros_agent

│ serial USB

▼

Teensy 4.1

│

Sensors → Simple Estimator → Safety/Mode

│                  │

└→ Controller → Wrench → TAM/Allocator → Backend

│

DISABLED / MOCK / future PWM

## 2.1 Konvensi penting

BODY frame: FRD — +X forward, +Y right/starboard, +Z down.

Depth positif ke bawah.

Manual command dan validation-stage wrench bersifat normalized/dimensionless pada rentang [-1, +1].

Simple estimator tidak menghasilkan XY position atau linear velocity.

Build production teensy41 tidak mempunyai production thruster geometry, sehingga allocator production sengaja belum ready.

# 3. Struktur workspace dan tanggung jawab

| Path | Fungsi |
| --- | --- |
| ~/tyrant-simple/firmware/teensy | Firmware Teensy 4.1, sensor, estimator sederhana, safety, controller, allocator, actuation, SysID, HIL state source. |
| ~/tyrant-simple/ros2_ws/src/tyrant_interfaces | Definisi message ROS 2 yang harus sama dengan copy interface yang dipakai micro-ROS firmware. |
| ~/tyrant-simple/ros2_ws/src/tyrant_teleop | Bridge controller/joystick → ManualCommand + heartbeat + mode/depth-reference request. |
| ~/tyrant-simple/ros2_ws/src/tyrant_sim | Reduced HIL plant + deterministic SysID excitation generator. |
| ~/tyrant-simple/companion/jetson | Helper script yang nantinya dipakai di companion computer/Jetson. |
| ~/tyrant-simple/docs | Architecture, TAM, SysID, teleop, mission/emergency, HIL, actuation notes. |

# 4. Preflight laptop

Baseline yang disarankan: Ubuntu 22.04 + ROS 2 Humble. Jalankan pengecekan berikut dari terminal baru.

cd ~/tyrant-simple

test -f firmware/teensy/platformio.ini && echo "platformio.ini OK"

test -f ros2_ws/src/tyrant_interfaces/package.xml && echo "tyrant_interfaces OK"

test -f ros2_ws/src/tyrant_teleop/package.xml && echo "tyrant_teleop OK"

test -f ros2_ws/src/tyrant_sim/package.xml && echo "tyrant_sim OK"

source /opt/ros/humble/setup.bash

ros2 --help >/dev/null && echo "ROS 2 Humble OK"

PASS gate: Jangan lanjut jika repository path salah atau `/opt/ros/humble/setup.bash` tidak ada. Selesaikan instalasi ROS 2 Humble lebih dulu.

# 5. Setup dependency ROS 2

sudo apt update

sudo apt install -y \

git unzip curl \

python3-venv python3-pip python3-pytest \

python3-colcon-common-extensions python3-rosdep \

ros-humble-launch-testing \

ros-humble-launch-testing-ament-cmake \

ros-humble-joy

Jika rosdep belum pernah diinisialisasi:

sudo rosdep init

rosdep update

Jika sudo rosdep init mengatakan sudah initialized, itu aman; cukup lanjut rosdep update.

## 5.1 Fix package.xml untuk pure-Python package

Pada snapshot repo yang dibuat sebelumnya, tyrant_teleop/package.xml dan tyrant_sim/package.xml masih memiliki <buildtool_depend>ament_python</buildtool_depend>. Pada setup lokal kita, rosdep tidak mempunyai key tersebut dan menghasilkan error “Cannot locate rosdep definition for [ament_python]”. Untuk kedua package pure-Python ini, hapus hanya baris buildtool dependency tersebut dan pertahankan export build type.

# HAPUS dari kedua package.xml:

<buildtool_depend>ament_python</buildtool_depend>

# TETAP PERTAHANKAN:

<export>

<build_type>ament_python</build_type>

</export>

Setelah diperbaiki:

cd ~/tyrant-simple/ros2_ws

source /opt/ros/humble/setup.bash

rosdep install --from-paths src --ignore-src -r -y

Error yang sudah dialami: Pesan rosdep tentang `ament_python` bukan error pada algoritma Tyrant; itu metadata package. Setelah baris tersebut dihapus, rosdep seharusnya bersih.

# 6. Build ROS workspace dari nol

## 6.1 Build interface lebih dulu

cd ~/tyrant-simple/ros2_ws

source /opt/ros/humble/setup.bash

colcon build \

--packages-select tyrant_interfaces \

--symlink-install

source install/setup.bash

ros2 interface show tyrant_interfaces/msg/SimpleState

ros2 interface show tyrant_interfaces/msg/ManualCommand

ros2 interface show tyrant_interfaces/msg/ThrusterCommand

ros2 interface show tyrant_interfaces/msg/ActuatorStatus

PASS gate: Empat interface di atas harus tampil. Jika ROS mengatakan message type invalid/tidak ditemukan, workspace belum dibuild atau `install/setup.bash` belum disource pada terminal tersebut.

## 6.2 Build teleop dan simulator

colcon build \

--packages-select tyrant_teleop tyrant_sim \

--symlink-install

source install/setup.bash

ros2 pkg list | grep '^tyrant_'

Expected:

tyrant_interfaces

tyrant_sim

tyrant_teleop

# 7. Isolasi Python ROS dari PlatformIO

PlatformIO memakai private Python virtual environment ~/.platformio/penv. Jangan menambahkan seluruh ~/.platformio/penv/bin di depan PATH pada mesin yang juga dipakai ROS, karena pytest milik PlatformIO dapat mengambil alih colcon test dan konflik dengan plugin ROS Humble launch_testing.

Error yang sudah dialami: PluginValidationError: Plugin launch_testing ... pytest_pycollect_makemodule(path, parent) dan traceback menunjuk ke ~/.platformio/penv/.../pytest.

# Hapus PATH global PlatformIO penv dari bashrc

sed -i '\|\.platformio/penv/bin|d' ~/.bashrc

# Expose hanya executable PlatformIO

mkdir -p ~/.local/bin

ln -sf ~/.platformio/penv/bin/pio ~/.local/bin/pio

ln -sf ~/.platformio/penv/bin/platformio ~/.local/bin/platformio

grep -qxF 'export PATH="$HOME/.local/bin:$PATH"' ~/.bashrc || \

echo 'export PATH="$HOME/.local/bin:$PATH"' >> ~/.bashrc

Untuk terminal yang sudah telanjur terkontaminasi:

export PATH="$(

echo "$PATH" |

tr ':' '\n' |

grep -v "$HOME/.platformio/penv/bin" |

paste -sd ':' -

)"

export PATH="$HOME/.local/bin:$PATH"

hash -r

which python3

which pytest

which pio

python3 -c "import pytest; print(pytest.__file__); print(pytest.__version__)"

Target: python3=/usr/bin/python3, pytest=/usr/bin/pytest, dan pio=~/.local/bin/pio. Python/pytest ROS tidak boleh menunjuk ke .platformio/penv.

# 8. ROS/Python regression tests

Jika sebelumnya test gagal karena pytest PlatformIO, bersihkan hanya package yang terdampak lalu build/test ulang.

cd ~/tyrant-simple/ros2_ws

rm -rf \

build/tyrant_sim build/tyrant_teleop \

install/tyrant_sim install/tyrant_teleop

source /opt/ros/humble/setup.bash

colcon build \

--packages-select tyrant_teleop tyrant_sim \

--symlink-install

source install/setup.bash

colcon test --packages-select tyrant_teleop tyrant_sim

colcon test-result --verbose

Baseline v1.2 mendefinisikan 8 pure-Python mapper tests pada tyrant_teleop dan 10 plant/profile tests pada tyrant_sim. Jalankan seluruh suite pada revisi aktif, termasuk regression test konfigurasi controller bila tersedia. Target lokal: zero errors dan zero failures; catat jumlah test aktual dari hasil runner.

# 9. Install dan cek PlatformIO

Jika pio --version belum bekerja, instal PlatformIO Core melalui installer resminya ke private environment, lalu expose hanya executable pio seperti pada bagian sebelumnya.

cd ~

curl -fsSL -o get-platformio.py \

https://raw.githubusercontent.com/platformio/platformio-core-installer/master/get-platformio.py

python3 get-platformio.py

mkdir -p ~/.local/bin

ln -sf ~/.platformio/penv/bin/pio ~/.local/bin/pio

ln -sf ~/.platformio/penv/bin/platformio ~/.local/bin/platformio

pio --version

## 9.1 USB/udev permission

curl -fsSL \

https://raw.githubusercontent.com/platformio/platformio-core/develop/platformio/assets/system/99-platformio-udev.rules \

| sudo tee /etc/udev/rules.d/99-platformio-udev.rules >/dev/null

sudo udevadm control --reload-rules

sudo udevadm trigger

sudo usermod -a -G dialout "$USER"

Logout/login atau reboot sekali setelah menambah group dialout.

# 10. Firmware build profiles

| Environment | Tujuan | Physical output |
| --- | --- | --- |
| teensy41 | Production-safe firmware. Sensor nyata + estimator + safety. Production geometry belum tersedia. | Tidak ada physical PWM; allocator production belum ready. |
| teensy41-debug | Production + diagnostic test topics /tyrant/test_command dan /tyrant/test_response. | Sama seperti production. |
| teensy41-mock-thrusters | End-to-end software command → synthetic TAM → mock 8 thruster. | Tidak ada PWM. |
| teensy41-hil | Mock thruster + ROS injected SimpleState dari reduced plant. | Tidak ada PWM/sensor fisik dibutuhkan untuk state control. |
| teensy41-test | Core embedded Unity regression tests. | Test firmware only. |
| teensy41-test-connection | ConnectionManager 4-state dengan ROS stub. | Test firmware only. |
| teensy41-hwt905-smoke | Standalone UART HWT905 smoke test. | Sensor only. |
| teensy41-fulldepth-smoke | Standalone I2C FullDepth/MS5837 smoke test. | Sensor only. |
| teensy41-pwm-skeleton | Compile physical PWM branch dengan pin development 0xFF. | Fail-closed; tidak attach Servo channel. |

## 10.1 Compile semua profile sebelum upload

cd ~/tyrant-simple/firmware/teensy

pio pkg install

pio run -e teensy41

pio run -e teensy41-mock-thrusters

pio run -e teensy41-hil

pio run -e teensy41-pwm-skeleton

pio run -e teensy41-hwt905-smoke

pio run -e teensy41-fulldepth-smoke

PASS gate: Jangan upload ke hardware sebelum seluruh profile yang relevan berhasil compile. Ini memisahkan error dependency/source dari error USB/hardware.

# 11. Embedded Teensy regression tests

Baseline repo terbaru mendefinisikan total 155 RUN_TEST cases. teensy41-test memuat core regression; teensy41-test-connection dipisahkan agar ConnectionManager dapat memakai stub ROS kecil.

cd ~/tyrant-simple/firmware/teensy

pio test -e teensy41-test

pio test -e teensy41-test-connection

Sub-suite yang penting untuk isolasi:

pio test -e teensy41-test -f test_simple_state_estimator

pio test -e teensy41-test -f test_manual_control

pio test -e teensy41-test -f test_thruster_allocation

pio test -e teensy41-test -f test_thruster_backend

pio test -e teensy41-test -f test_sysid_recorder

pio test -e teensy41-test -f test_sim_state_source

pio test -e teensy41-test-connection -f test_connection_manager

Penting: Sesudah `pio test`, firmware test tertinggal di Teensy. Upload ulang firmware production/mock/HIL yang ingin dipakai.

# 12. Hardware binding dan sensor smoke tests

## 12.1 HWT905

Production binding saat ini: Serial1, baud 9600, verified mounting [+X, -Y, -Z]. Binding ada di src/sensors/sensor_hardware.cpp, parameter di include/config/sensor_config.h.

pio run -e teensy41-hwt905-smoke -t upload

pio device monitor -b 115200

PASS: frame terbaca stabil, accel/gyro/mag masuk akal. Setelah smoke PASS, upload ulang production.

## 12.2 FullDepth / MS5837

Production binding memakai default Wire, I2C 100 kHz, sample period 200 ms (5 Hz), dan default water density 1025 kg/m³. Untuk freshwater tank dapat sementara memakai sekitar 997 kg/m³.

pio run -e teensy41-fulldepth-smoke -t upload

pio device monitor -b 115200

PASS: pressure atmosfer masuk akal, temperature masuk akal, pressure bertambah saat sensor diturunkan lebih dalam.

# 13. Upload firmware production

cd ~/tyrant-simple/firmware/teensy

pio run -e teensy41 -t clean

pio run -e teensy41

pio run -e teensy41 -t upload

Build production sengaja tidak mengaktifkan thruster geometry/PWM. Output thruster zero/fail-closed adalah expected sampai hardware configuration final diisi.

# 14. micro-ROS agent: reuse workspace lama dan reconnect

micro-ROS agent yang sudah pernah dibuild di workspace tyrant-auv boleh direuse. Agent adalah host-side executable; tidak harus berada di repository tyrant-simple. Syarat utama: agent dibuild untuk ROS 2 Humble dan firmware Teensy menggunakan distro Humble (board_microros_distro = humble).

Contoh jika agent lama berada di ~/tyrant-auv/microros_ws (sesuaikan dengan path aktual):

source /opt/ros/humble/setup.bash

source ~/tyrant-auv/microros_ws/install/local_setup.bash

ros2 run micro_ros_agent micro_ros_agent \

serial --dev /dev/ttyACM0 -v6

Pada terminal project:

source /opt/ros/humble/setup.bash

source ~/tyrant-simple/ros2_ws/install/setup.bash

ros2 topic list

## 14.1 ConnectionManager 4-state

Firmware mempertahankan state: WAITING_AGENT → AGENT_AVAILABLE → CONNECTED → DISCONNECTED. Ping agent dilakukan sekitar setiap 500 ms dengan timeout 50 ms. Saat agent hilang, transport health langsung false, ROS entities dihancurkan, lalu firmware kembali menunggu agent. Reconnect harus terjadi tanpa reset Teensy.

| State | Makna |
| --- | --- |
| WAITING_AGENT (0) | Teensy hidup, agent belum terlihat; ping periodik. |
| AGENT_AVAILABLE (1) | Agent ditemukan; firmware mencoba membuat ROS entities. |
| CONNECTED (2) | Executor spin + periodic ping aktif; topic berjalan. |
| DISCONNECTED (3) | Ping gagal; entities dihancurkan lalu kembali WAITING_AGENT. |

## 14.2 Reconnect acceptance test

Boot Teensy sementara micro-ROS agent OFF. Jangan reset board.

Start agent. Topic Tyrant harus muncul otomatis.

Stop agent dengan Ctrl+C. Tunggu firmware mendeteksi disconnect.

Start agent lagi. Topic harus kembali tanpa reset Teensy.

Ulangi minimal 5–10 kali. Pantau `/tyrant/communication/diagnostics` untuk connection_count, disconnect_count, reconnect_count.

Praktik bagus: Jika device name `/dev/ttyACM0` berubah setelah reconnect/reboot, gunakan stable path di `/dev/serial/by-id/` bila tersedia.

## 14.3 Setup terminal khusus micro-ROS validation

Gunakan minimal tiga terminal agar agent, monitoring, dan command tidak saling menimpa environment.

Terminal A — micro-ROS Agent

source /opt/ros/humble/setup.bash

source ~/microros_ws/install/local_setup.bash

ls -l /dev/serial/by-id/

ros2 run micro_ros_agent micro_ros_agent \

serial \

--dev /dev/serial/by-id/<NAMA_TEENSY> \

-v6

Gunakan `/dev/serial/by-id/...` bila tersedia karena lebih stabil daripada `/dev/ttyACM0`, yang dapat berubah menjadi `ttyACM1` setelah reconnect/reboot.

Terminal B — monitoring project ROS

source /opt/ros/humble/setup.bash

source ~/tyrant-simple/ros2_ws/install/setup.bash

Terminal C — command/test

source /opt/ros/humble/setup.bash

source ~/tyrant-simple/ros2_ws/install/setup.bash

Jangan jalankan `pio device monitor` pada port serial yang sama saat micro-ROS agent aktif. Setelah `pio test`, upload ulang firmware production karena test firmware tertinggal di Teensy.

## 14.4 micro-ROS validation matrix

| Test | Yang diuji | PASS criteria |
| --- | --- | --- |
| Entity creation | Node dan seluruh ROS entities dapat dibuat | `/tyrant_teensy` muncul dan topic Tyrant lengkap; session tidak langsung ditutup. |
| Teensy → ROS | Publisher path | IMU/state/health berubah dan sequence/timestamp terus naik. |
| ROS → Teensy | Subscriber path | Heartbeat, mode request, dan ManualCommand memengaruhi state firmware. |
| Topic rate | Scheduler + transport | Rate stabil; tidak berhenti periodik atau jatuh ke 0. |
| Manual timeout | Command freshness | Manual publish berhenti >250 ms → wrench zero; heartbeat tetap healthy. |
| Heartbeat timeout | Safety authority | Heartbeat berhenti >500 ms → communication unhealthy, SAFE, zero wrench. |
| Agent restart | ConnectionManager lifecycle | Agent restart → reconnect tanpa reset Teensy. |
| Boot order | Startup robustness | Agent-first dan Teensy-first sama-sama berhasil connect. |
| Long run | Session stability | 30 menit tanpa create/destroy loop atau session collapse. |
| Stress ringan | Entity/resource headroom | Beberapa echo/hz + command publisher aktif bersamaan tanpa disconnect. |

## 14.5 Test entity creation dan ROS graph

Setelah agent dan production firmware aktif:

ros2 node list

ros2 topic list | grep tyrant

PASS minimum:

/tyrant_teensy

Topic utama yang perlu terlihat mencakup sensor, state, health, mode, control, actuation, mission/emergency, dan SysID sesuai build profile. Jika node muncul tetapi hanya sebagian topic, curigai kegagalan `createEntities()` atau micro-ROS resource capacity.

## 14.6 Test Teensy → ROS publisher path

ros2 topic echo /tyrant/sensors/imu

ros2 topic echo /tyrant/simple_state

ros2 topic echo /tyrant/sensors/pressure

ros2 topic echo /tyrant/system/health

ros2 topic echo /tyrant/mode/status

Yang dicek:

IMU `sequence` dan `timestamp_us` terus naik.

IMU `valid=true`, `sensor_state=READY`, error counter tidak naik terus-menerus.

`simple_state` bounded dan validity flags sesuai sensor.

Pressure/depth masuk akal dan tidak stale.

System/mode telemetry tetap terbit selama session aktif.

## 14.7 Test topic rate dan bandwidth

ros2 topic hz /tyrant/sensors/imu

ros2 topic hz /tyrant/simple_state

ros2 topic hz /tyrant/system/health

ros2 topic bw /tyrant/sensors/imu

Fokus awal bukan angka presisi, melainkan rate stabil, tidak berhenti periodik, tidak menjadi 0, dan agent tidak masuk create/destroy loop. Bandingkan rate aktual dengan scheduler/telemetry cadence pada Section 15.

## 14.8 Test ROS → Teensy subscriber path

Jalankan host heartbeat:

ros2 topic pub -r 10 \

/tyrant/host_heartbeat \

std_msgs/msg/UInt32 \

"{data: 1}"

Request MANUAL:

ros2 topic pub --once \

/tyrant/mode/request \

tyrant_interfaces/msg/ModeRequest \

"{request_id: 100, requested_mode: 2}"

Monitor:

ros2 topic echo /tyrant/mode/status

ros2 topic echo /tyrant/system/health

PASS: communication healthy, MANUAL diterima bila readiness/safety memungkinkan, dan propulsion authority sesuai policy build saat itu.

Kemudian kirim ManualCommand:

ros2 topic pub -r 20 \

/tyrant/manual/command \

tyrant_interfaces/msg/ManualCommand \

"{sequence: 1, surge: 0.2, sway: 0.0, heave: 0.0, roll: 0.0, pitch: 0.0, yaw: 0.0}"

ros2 topic echo /tyrant/control/wrench_request

Pada production tanpa geometry aktual, `thruster_command` boleh tetap zero/invalid; yang divalidasi di sini adalah subscriber → controller → wrench path.

## 14.8.1 ManualCommand sequence, state validity, dan sensorless test

Catatan penting: `ros2 topic pub -r` mengulang payload yang sama, termasuk field `sequence`. Pada firmware yang diuji 2026-10-07, sequence statis tetap dapat menghasilkan `command_fresh: true`, jadi sequence statis tidak boleh dipakai sendiri sebagai diagnosis stale. Untuk test yang lebih representatif terhadap teleop nyata, gunakan `tyrant_teleop` atau publisher yang menaikkan `sequence` setiap paket.

Freshness ditentukan oleh umur penerimaan command valid di Teensy, bukan perubahan sequence. Untuk meniru teleop, gunakan publisher dengan sequence meningkat; untuk membuktikan timeout, hentikan publisher dan pastikan wrench zero setelah lebih dari 250 ms. Sebelum menjalankan publisher Python, pastikan ROS memakai Python sistem, bukan Python private PlatformIO:

which python3

# target: /usr/bin/python3

python3 -c "import numpy; print(numpy.__version__)"

python3 -c "from tyrant_interfaces.msg import ManualCommand; print('ManualCommand import OK')"

Jika `which python3` menunjuk ke `~/.platformio/penv/bin/python3`, keluarkan `.platformio/penv/bin` dari PATH dan expose hanya `pio` melalui `~/.local/bin/pio`. Jika NumPy belum tersedia pada Python sistem, install `python3-numpy`.

Interpretasi WrenchRequest

Jika `source_sequence` sudah non-zero/berubah dan `command_fresh: true`, maka micro-ROS, subscription, executor callback, dan ManualCommandManager sudah bekerja. Jika pada saat yang sama `enabled: false` dan seluruh wrench tetap zero, lanjutkan diagnosis ke state/controller safety gate—bukan kembali mengubah micro-ROS.

ros2 topic echo /tyrant/simple_state --once

ros2 topic echo /tyrant/sensors/imu --once

ros2 topic echo /tyrant/sensors/pressure --once

`updateStabilizedManualController()` sengaja fail-closed jika command stale, propulsion tidak diizinkan, control dt invalid, state FAULT/non-finite, atau `state.attitude_valid == false`.

Production firmware membutuhkan sensor nyata

Pada profile `teensy41`, state attitude berasal dari pipeline HWT905. Jika HWT905 memang tidak dipasang, `bytes_received=0`, `valid=false`, `attitude_valid=false`, dan wrench `enabled=false` adalah expected behavior. Ini bukan kegagalan micro-ROS.

FullDepth yang tidak dipasang juga membuat pressure/depth invalid. Depth validity dibutuhkan oleh depth-hold/AUTO yang memerlukannya, walaupun blocker langsung stabilized manual controller saat ini adalah attitude validity.

Jangan memaksa validity flag menjadi true pada production hanya untuk membuat controller aktif. Pilih jalur test yang sesuai:

Production hardware test: validasi HWT905 di bench, kemudian upload kembali teensy41. FullDepth smoke test dan validasi reference/depth wajib setelah sensor fisik tersedia dan sebelum kontrol depth fisik; sementara itu, gunakan HIL untuk jalur software yang membutuhkan depth.

Sensorless control test: upload `teensy41-hil` dan jalankan `tyrant_plant_sim`; state valid diinjeksikan melalui `/tyrant/sim/state_input` tanpa sensor fisik.

Flow sensorless HIL:

ManualCommand / Mission / SysID

↓

Teensy real controller + allocator

↓

mock thruster command

↓

tyrant_plant_sim → /tyrant/sim/state_input → Teensy HIL state source

## 14.9 Test manual-command timeout dan heartbeat timeout

Manual timeout: heartbeat tetap hidup, hentikan hanya publisher `/tyrant/manual/command`. Dalam sekitar 250 ms:

command_fresh: false

enabled: false

X Y Z K M N -> 0

Communication harus tetap healthy. Ini membuktikan manual-command freshness terpisah dari heartbeat.

Heartbeat timeout: setelah kembali MANUAL, hentikan `/tyrant/host_heartbeat`. Dalam sekitar 500 ms:

communication_healthy: false

mode -> SAFE

propulsion_allowed: false

wrench -> 0

Test ini wajib PASS sebelum actuator fisik diaktifkan.

## 14.10 Auto-reconnect tanpa reset Teensy

Dengan Teensy CONNECTED, hentikan agent (`Ctrl+C`) tanpa mereset Teensy. Tunggu sekitar 3–5 detik, lalu start agent lagi dengan command yang sama.

CONNECTED

-> DISCONNECTED

-> WAITING_AGENT

-> AGENT_AVAILABLE

-> CONNECTED

`ros2 node list` harus kembali menampilkan `/tyrant_teensy` tanpa board reset. Ulangi minimal 10 kali. Pantau `/tyrant/communication/diagnostics` bila tersedia dan pastikan reconnect counter bertambah secara konsisten.

## 14.11 Boot-order test

1. Agent ON → Teensy boot → connect.

2. Teensy boot sementara agent OFF → firmware menunggu → agent ON → connect otomatis.

Skenario kedua penting untuk deployment Jetson karena companion computer dapat reboot sementara Teensy tetap hidup.

## 14.12 Long-run dan stress test

Jalankan session minimal 30 menit sambil memonitor:

ros2 topic hz /tyrant/simple_state

watch -n 1 'ros2 node list'

Di beberapa terminal, jalankan subscriber/publisher bersamaan:

ros2 topic echo /tyrant/simple_state

ros2 topic hz /tyrant/sensors/imu

ros2 topic echo /tyrant/system/health

ros2 topic echo /tyrant/communication/diagnostics

Heartbeat dan ManualCommand boleh dijalankan bersamaan. Tidak boleh muncul pola berulang:

delete_client

session closed

create/destroy loop

topic hilang-muncul terus

## 14.13 Interpretasi cepat log micro-ROS agent

| Log/pola | Interpretasi | Langkah berikutnya |
| --- | --- | --- |
| Hanya `running...`, `logger setup...` | Agent membuka serial tetapi client belum handshake. | Pastikan firmware production aktif, port benar, dan port tidak dipakai proses lain. |
| `create_session` + create participant/topic/pub/sub | XRCE handshake dan entity creation berlangsung. | Cek ROS graph dan topic. |
| Session lalu cepat banyak `delete_object` + `session closed` | Biasanya `createEntities()` gagal lalu firmware cleanup. | Cek resource limit publisher/subscriber, message support, dan entity create return code. |
| Traffic normal kontinu | Session sehat. | Lanjut rate/reconnect/long-run test. |
| Reconnect hanya setelah reset Teensy | Lifecycle client/ConnectionManager belum robust. | Debug destroy/recreate path; reset bukan acceptance workaround. |

## 14.14 micro-ROS resource dan API compatibility checks

`tyrant-simple` mempunyai entity lebih banyak daripada default micro-ROS profile. Pastikan custom meta menyediakan headroom, misalnya baseline development:

RMW_UXRCE_MAX_PUBLISHERS >= 20

RMW_UXRCE_MAX_SUBSCRIPTIONS >= 12

Setelah mengubah user meta/resource, rebuild library micro-ROS, bukan hanya object firmware:

cd ~/tyrant-simple/firmware/teensy

pio run -e teensy41 -t clean_microros

pio run -e teensy41 -t clean

pio run -e teensy41

pio run -e teensy41 -t upload

Untuk destroy-session timeout, API micro-ROS membutuhkan `rmw_context_t*`; ambil dari `rcl_context_t` dengan `rcl_context_get_rmw_context(&support.context)`. Hindari cast paksa.

# 15. Scheduler dan timeout kritis

| Item | Nilai baseline | Efek |
| --- | --- | --- |
| Control task | 20 ms / 50 Hz | Update stabilized control + allocator/backend path. |
| Safety task | 100 ms / 10 Hz | Safety state enforcement. |
| Health telemetry | 200 ms / 5 Hz | State/sensor/actuator/system status telemetry. |
| Heartbeat + comm diagnostics | 1000 ms / 1 Hz | Teensy heartbeat and connection diagnostics. |
| Host heartbeat timeout | 500 ms | MANUAL/AUTO dipaksa SAFE jika communication unhealthy. |
| Manual command timeout | 250 ms | Stale manual command menghasilkan zero wrench. |
| Actuator backend watchdog | 100 ms | Stale enabled actuator command dinetralkan + fault latch. |
| HIL state freshness | 200 ms | State HIL stale menghapus valid flags dan menutup control/autonomy gates. |
| IMU stale timeout | 500 ms | IMU dianggap stale. |
| Pressure stale timeout | 1000 ms | Pressure dianggap stale. |

# 16. ROS nodes dan fungsi

| Node / process | Package / lokasi | Fungsi utama |
| --- | --- | --- |
| micro_ros_agent | Workspace agent lama / micro-ROS | Bridge serial XRCE-DDS antara Teensy dan ROS 2 graph. |
| tyrant_teensy | Firmware micro-ROS node | Semua interface kendaraan: state, sensors, safety/mode, control, allocator, actuator, SysID, mission/emergency. |
| joy_node | ROS package joy | Membaca Linux joystick/controller dan publish sensor_msgs/Joy ke /joy. |
| tyrant_joy_probe | tyrant_teleop | Calibration helper: menunjukkan perubahan axis/button untuk mapping controller. |
| tyrant_teleop | tyrant_teleop | Mapping /joy → ManualCommand; deadman; heartbeat; MANUAL/SAFE/depth-reference button actions. |
| tyrant_plant_sim | tyrant_sim | HIL synthetic reduced plant: ThrusterCommand → synthetic wrench → depth/attitude/rates → SimpleState. |
| tyrant_sysid_excitation | tyrant_sim | Deterministic step/PRBS/chirp command generator; heartbeat + MANUAL handshake + SysID START/STOP + SAFE on finish. |

# 17. Topic map — publisher dari Teensy

| Topic | Type | Fungsi / expected consumer |
| --- | --- | --- |
| /tyrant/heartbeat | std_msgs/UInt32 | Heartbeat Teensy; host/GCS dapat memonitor liveness. |
| /tyrant/mode/status | ModeStatus | Current/requested mode, acceptance reason, communication health, propulsion authority. |
| /tyrant/system/health | SystemHealth | Ringkasan communication, estimator readiness, autonomy readiness, propulsion authority. |
| /tyrant/simple_state | SimpleState | Reduced state: roll/pitch/yaw, p/q/r, yaw bias, depth, validity flags. |
| /tyrant/sensors/imu | ImuTelemetry | Raw mounted IMU + magnetometer + driver health/stats. |
| /tyrant/sensors/pressure | PressureTelemetry | FullDepth pressure/temp + source health/stats. |
| /tyrant/depth/reference/status | DepthReferenceStatus | Ack/reason/result surface-reference request. |
| /tyrant/communication/diagnostics | CommunicationDiagnostics | 4-state connection and reconnect/error counters. |
| /tyrant/control/wrench_request | WrenchRequest | Normalized BODY-FRD X/Y/Z/K/M/N request setelah controller. |
| /tyrant/control/thruster_command | ThrusterCommand | 8 normalized commands dari allocator + readiness/valid/saturation/residual. |
| /tyrant/actuation/status | ActuatorStatus | Backend readiness/fault/watchdog/pulse output diagnostic. |
| /tyrant/sysid/status | SysIdStatus | Session state/IDs/sample sequence. |
| /tyrant/sysid/sample | SysIdSample | Synchronized command/wrench/thruster/state/raw-sensor sample untuk model identification. |
| /tyrant/mission/status | MissionStatus | Mission state, reason, step index/type. |
| /tyrant/emergency/status | EmergencyStatus | Recovery policy/state/reason/elapsed time/actuation request. |
| /tyrant/test_response | std_msgs/UInt32 | Hanya build debug; response diagnostics. |

# 18. Topic map — subscriber Teensy

| Topic | Type | Producer umum / fungsi |
| --- | --- | --- |
| /tyrant/mode/request | ModeRequest | GCS/teleop/SysID: request IDLE/MANUAL/AUTO/SAFE. |
| /tyrant/host_heartbeat | std_msgs/UInt32 | Teleop/supervisor/SysID. Harus fresh untuk MANUAL/AUTO. |
| /tyrant/depth/reference/request | DepthReferenceRequest | Set/clear surface pressure reference. |
| /tyrant/manual/command | ManualCommand | Teleop/SysID/host command normalized. |
| /tyrant/sysid/control | SysIdControl | START/STOP synchronized SysID session. |
| /tyrant/mission/command | MissionCommand | Clear/append/start/stop/status mission steps. |
| /tyrant/emergency/control | EmergencyControl | Trigger zero-thrust atau controlled surface. |
| /tyrant/sim/state_input | SimpleState | Hanya teensy41-hil; state dari tyrant_plant_sim. |
| /tyrant/test_command | std_msgs/UInt32 | Hanya debug build. |

# 19. Host-side topic flow

## 19.1 PS controller / teleop

controller → joy_node → /joy

→ tyrant_teleop

├─ /tyrant/manual/command

├─ /tyrant/host_heartbeat

├─ /tyrant/mode/request

└─ /tyrant/depth/reference/request

Default teleop behavior: ManualCommand hanya dipublish saat deadman ditahan. Deadman release mengirim satu neutral command lalu berhenti publish agar 250 ms freshness timeout Teensy dapat menutup wrench. /joy stale 0.20 s menghentikan command; stale 1.00 s dapat request SAFE.

## 19.2 HIL plant

Teensy /tyrant/control/thruster_command

↓

tyrant_plant_sim

↓

/tyrant/sim/state_input

↓

Teensy HIL state source

## 19.3 SysID excitation

tyrant_sysid_excitation

├─ heartbeat

├─ repeated MANUAL request until confirmed

├─ SysID START

├─ deterministic ManualCommand profile

├─ neutral post-roll

├─ SysID STOP

└─ SAFE request

# 20. Configuration map

| File | Apa yang dikonfigurasi / kapan diubah |
| --- | --- |
| firmware/teensy/platformio.ini | Build profile, micro-ROS distro/transport, lib deps, test source filters, HIL/mock/PWM flags. |
| include/config/sensor_config.h | HWT905 baud/mounting/stale timeout; FullDepth I2C/sample period/water density/stale timeout. |
| src/sensors/sensor_hardware.cpp | Hardware binding: HWT905=Serial1; FullDepth=Wire. |
| include/config/simple_estimator_config.h | Stationary init, gravity tolerance, max dt, complementary time constant, 2-state yaw KF noises/timeouts. |
| include/config/control_config.h | Manual timeout, depth-rate, attitude reference limits, provisional P/PD gains, control max dt. |
| include/config/thruster_config.h | Backend watchdog, placeholder pulse mapping, allocator damping, production/synthetic geometry. |
| include/config/mission_config.h | Emergency controlled-surface target/tolerance/ascent command/timeout. |
| ros2_ws/src/tyrant_teleop/config/ps_controller.yaml | Controller axis/button mapping, deadzone, publish/heartbeat rates, joy SAFE timeout. |
| ros2_ws/src/tyrant_sim/config/reduced_plant.yaml | Synthetic HIL dynamics only; bukan hydrodynamic model final. |

# 21. Vehicle mode dan safety behavior

| Mode | Propulsion / behavior |
| --- | --- |
| BOOT | Initialization. |
| IDLE | No propulsion authority. |
| MANUAL | Propulsion allowed hanya jika communication healthy. |
| AUTO | Propulsion allowed hanya jika communication healthy + autonomy_ready. |
| SAFE | Zero propulsion authority; bukan position hold dan bukan automatic surface. |
| FAULT | Fail-closed/latched fault semantics. |
| EMERGENCY | Dedicated emergency actuation authority; policy saat ini zero-thrust atau controlled-surface. |

Host heartbeat timeout adalah 500 ms. Jika MANUAL/AUTO kehilangan transport atau heartbeat, safety memaksa SAFE. SAFE saat ini berarti no propulsion; kendaraan masih dapat hanyut/naik/turun pasif sesuai buoyancy/arus.

# 22. Local functional validation setelah production firmware hidup

## 22.1 Sanity ROS graph

ros2 node list

ros2 topic list | sort

ros2 node info /tyrant_teensy

Pastikan node dan topic utama muncul. Jika node tidak muncul, fokus dulu ke micro-ROS agent/serial/reconnect; jangan debug controller.

## 22.2 Sensor topics

ros2 topic echo /tyrant/sensors/imu

ros2 topic echo /tyrant/sensors/pressure

ros2 topic echo /tyrant/simple_state

ros2 topic echo /tyrant/system/health

IMU PASS: sensor_state READY, valid=true, error_count=0, samples_dropped tidak meningkat terus. Pressure PASS: READY/valid, pressure/temp masuk akal. Simple state PASS: roll/pitch/yaw bounded, angular velocity dekat nol saat diam, depth valid setelah reference diset.

## 22.3 Set surface depth reference

ros2 topic pub --once \

/tyrant/depth/reference/request \

tyrant_interfaces/msg/DepthReferenceRequest \

"{request_id: 1, command: 1}"

ros2 topic echo /tyrant/depth/reference/status

Request hanya diterima sesuai policy manager (terutama keadaan kendaraan/sensor pressure). Setelah reference valid, depth di posisi reference harus mendekati 0.

## 22.4 Host heartbeat + MANUAL software test

# Terminal A

ros2 topic pub -r 10 \

/tyrant/host_heartbeat std_msgs/msg/UInt32 "{data: 1}"

# Terminal B

ros2 topic pub --once \

/tyrant/mode/request tyrant_interfaces/msg/ModeRequest \

"{request_id: 10, requested_mode: 2}"

# Terminal C

ros2 topic echo /tyrant/mode/status

MANUAL accepted hanya jika communication healthy. Tanpa heartbeat fresh, request harus ditolak atau mode akhirnya dipaksa SAFE.

# 23. PS controller test

source /opt/ros/humble/setup.bash

source ~/tyrant-simple/ros2_ws/install/setup.bash

# Calibrate mapping

ros2 run joy joy_node

ros2 run tyrant_teleop tyrant_joy_probe

Gerakkan satu axis/button per waktu dan edit ps_controller.yaml. Setelah mapping benar:

ros2 launch tyrant_teleop ps_teleop.launch.py

ros2 topic echo /joy

ros2 topic echo /tyrant/manual/command

ros2 topic echo /tyrant/control/wrench_request

Deadman wajib. Jika stick bergerak tetapi ManualCommand tidak keluar: cek deadman index, joy_timeout, mapping index, dan source install/setup.bash setelah rebuild YAML/package.

### 23.1 Bluetooth controller bring-up — hasil aktual 2026-10-07

Controller PS yang diuji berhasil digunakan melalui Bluetooth. Selama Linux/ROS menerbitkan sensor_msgs/Joy di /joy, tyrant_teleop tidak membedakan transport USB vs Bluetooth.

Flow yang sudah tervalidasi:

PS controller (Bluetooth)

↓

Linux input → joy_node → /joy

↓

tyrant_teleop + ps_controller.yaml

↓

/tyrant/manual/command → Teensy / MANUAL

↓

/tyrant/control/wrench_request

Node yang diharapkan: /joy_node, /tyrant_joy_probe (optional/calibration), /tyrant_teleop, /tyrant_teensy.

Actual launch command yang berhasil:

source /opt/ros/humble/setup.bash

source ~/tyrant-simple/ros2_ws/install/setup.bash

ros2 run tyrant_teleop tyrant_teleop \

--ros-args \

--params-file ~/tyrant-simple/ros2_ws/src/tyrant_teleop/config/ps_controller.yaml

Jangan memakai placeholder NAMA_FILE.yaml. Setelah rebuild package, source install/setup.bash lagi.

### 23.2 Mapping controller: physical input vs fungsi Tyrant

| Input controller | Mapping baseline dan evidence | Fungsi di Tyrant |
| --- | --- | --- |
| Left stick vertical | axis[1] terkonfirmasi; up pernah terbaca sekitar +1.0 | Surge maju/mundur → ManualCommand.surge → wrench X. |
| Left stick horizontal | Index mengikuti ps_controller.yaml; verifikasi probe | Sway kiri/kanan → ManualCommand.sway → wrench Y. |
| Right stick horizontal | Index mengikuti YAML | Yaw/heading-rate; neutral dapat menjadi heading hold jika heading valid. |
| L2 / R2 analog trigger | Axis/normalization mengikuti YAML | Heave/depth-rate. Production membutuhkan depth valid untuk depth behavior yang benar. |
| L1 — deadman | button[4] pada baseline PRD/default; verifikasi probe | Harus ditahan untuk motion command. Release → neutral sekali lalu stop publish; Teensy timeout ~250 ms menutup wrench. |
| Cross — MANUAL | button[0] pada baseline PRD/default | Request MANUAL via /tyrant/mode/request; Teensy tetap accept/reject authority. |
| Square — IDLE | button[2] pada baseline PRD/default | Request IDLE via /tyrant/mode/request; tidak memberi propulsion authority. |
| Circle — SAFE | button[1] pada baseline PRD/default | Request SAFE; propulsion authority false dan wrench zero. |
| Triangle — surface reference | button[3] pada baseline PRD/default | Request depth/surface reference; butuh pressure/depth sensor valid dan policy mengizinkan. |

Untuk melihat mapping aktual yang sedang dipakai:

grep -nEi 'axis|button|deadman|manual|safe|surface|depth|surge|sway|yaw|heave|roll|pitch' \

~/tyrant-simple/ros2_ws/src/tyrant_teleop/config/ps_controller.yaml

Mapping fungsi tombol berikut mengikuti PRD V0.3 dan default teleop_node. Verifikasi index dengan tyrant_joy_probe, satu input pada satu waktu, karena Bluetooth/USB/driver dapat memberi urutan berbeda. Jika index berbeda, sesuaikan YAML sambil mempertahankan fungsi tombol. Mapping axis tetap mengikuti YAML dan hasil probe perangkat.

Catatan evidence v1.2: Square terdeteksi sebagai button[2] pada controller yang diuji. Catatan ini mengidentifikasi index fisik; fungsi Square pada baseline sekarang adalah IDLE. Deadman memakai L1 dan tetap memerlukan validasi perangkat. Tombol yang tidak dipetakan tidak menghasilkan aksi Tyrant.

### 23.3 Arti setiap fungsi input teleop

Surge: direct normalized manual effort ke BODY X; tidak memerlukan depth sensor.

Sway: direct normalized manual effort ke BODY Y; tidak memerlukan depth sensor.

Heave: depth-rate/depth-hold; bukan sekadar direct Z. Production memerlukan depth_valid untuk behavior depth yang benar.

Roll/Pitch: attitude reference terbatas; neutral berarti stabilisasi ke reference/level, jadi K/M dapat nonzero walau command 0.

Yaw: heading-rate command; neutral berarti heading hold, jadi N dapat nonzero untuk koreksi.

Deadman: host-side safety enable; tidak membypass heartbeat, mode gate, state validity, atau Teensy watchdog.

MANUAL/IDLE/SAFE: hanya request mode; Teensy tetap safety authority.

Surface reference: helper depth reference; bukan motion control dan tidak berguna tanpa pressure/depth valid.

### 23.4 Catatan acceptance teleop dan validasi ulang

Tabel berikut mempertahankan hasil sesi v1.2. Setelah sinkronisasi YAML, ulangi probe dan acceptance test dengan L1 sebagai deadman: Cross meminta MANUAL, Square IDLE, Circle SAFE, dan Triangle surface reference saat sensor/policy memungkinkan. Status PASS lama tidak membuktikan mapping revisi ini telah diuji pada controller fisik.

ros2 topic echo /joy

ros2 topic echo /tyrant/manual/command

ros2 topic echo /tyrant/control/wrench_request

| Validation item | Status sesi v1.2 |
| --- | --- |
| Bluetooth controller → /joy | PASS |
| tyrant_joy_probe axis/button | PASS |
| tyrant_teleop + ps_controller.yaml | PASS |
| Controller → /tyrant/manual/command | PASS |
| Real HWT905 + MANUAL + fresh command → wrench enabled | PASS |
| Surge path | PASS |
| Sway + clamp/saturation behavior | PASS |
| Manual-command timeout ~250 ms → zero wrench | PASS |
| Host-heartbeat timeout ~500 ms → SAFE + zero wrench | PASS |
| micro-ROS restart → reconnect without Teensy reset | PASS |
| Heave production with real FullDepth | DEFERRED — depth sensor belum dapat dipasang |
| Dedicated deadman-release / Bluetooth-disconnect SAFE test | PENDING explicit evidence |

# 24. Mock thruster test tanpa motor

cd ~/tyrant-simple/firmware/teensy

pio run -e teensy41-mock-thrusters -t upload

ros2 topic echo /tyrant/control/wrench_request

ros2 topic echo /tyrant/control/thruster_command

ros2 topic echo /tyrant/actuation/status

Expected mock build: allocator_ready=true, valid=true saat command aktif, mock_backend=true, delapan command virtual berubah. Deadman/command timeout/SAFE harus mengembalikan delapan channel ke zero.

# 25. HIL closed-loop test

Gunakan HIL ketika ingin menguji micro-ROS, ManualCommand, stabilized controller, allocator, mission, emergency, atau SysID tanpa memasang HWT905 dan FullDepth fisik. Production `teensy41` tidak diharapkan menghasilkan controller enabled jika state sensor memang tidak tersedia.

cd ~/tyrant-simple/firmware/teensy

pio run -e teensy41-hil -t clean

pio run -e teensy41-hil

pio run -e teensy41-hil -t upload

Setelah upload HIL, biarkan micro-ROS agent tetap aktif dan jalankan simulator dari terminal ROS project. Verifikasi `/tyrant/sim/state_input` dan `/tyrant/simple_state` sebelum mengirim command.

# terminal ROS

source /opt/ros/humble/setup.bash

source ~/tyrant-simple/ros2_ws/install/setup.bash

ros2 launch tyrant_sim hil_plant.launch.py

ros2 topic echo /tyrant/sim/state_input

ros2 topic echo /tyrant/simple_state

ros2 topic echo /tyrant/control/thruster_command

HIL PASS: plant state dan Teensy simple_state konsisten, signs depth/yaw benar, state tidak runaway untuk neutral input. Stop plant >200 ms harus meng-invalid-kan HIL state dan menutup controller/autonomy gates.

Batas validitas: Reduced plant HIL adalah synthetic development plant. Data HIL tidak boleh dipakai sebagai final LMPC vehicle model.

HIL acceptance tambahan:

`/tyrant/sim/state_input` terbit dan Teensy menerima state sebelum ManualCommand diuji.

`attitude_valid`, `heading_valid`, dan `depth_valid` mengikuti state simulator.

Dengan heartbeat + MANUAL + command fresh dan seluruh safety gate terpenuhi, WrenchRequest dapat `enabled=true`.

Stop plant >200 ms harus meng-invalid-kan HIL state dan menutup controller/autonomy gates.

Profile HIL tidak mengaktifkan PWM fisik.

Setelah selesai HIL dan kembali ke sensor nyata, upload ulang production:

cd ~/tyrant-simple/firmware/teensy

pio run -e teensy41 -t clean

pio run -e teensy41

pio run -e teensy41 -t upload

# 26. SysID test

Contoh automatic HIL SysID yaw step:

ros2 run tyrant_sim tyrant_sysid_excitation --ros-args \

-p experiment_id:=1001 \

-p profile:=step_yaw \

-p amplitude:=0.25 \

-p duration_s:=12.0

ros2 topic echo /tyrant/sysid/status

ros2 topic echo /tyrant/sysid/sample

SysID node melakukan heartbeat, MANUAL handshake, START session, pre-roll, excitation, post-roll, STOP, lalu request SAFE. Jika MANUAL tidak established dalam timeout atau kendaraan keluar MANUAL, experiment abort ke neutral/SAFE.

# 27. Mission dan emergency local test

AUTO production normal masih expected blocked sampai real geometry + backend + valid reduced state + mission readiness tersedia. Untuk software test gunakan mock/HIL profile.

Emergency controlled-surface tetap development logic. Jika depth/attitude invalid atau timeout, recovery fail-closed ke zero thrust.

# 28. Troubleshooting matrix

| Gejala / error | Penyebab umum | Tindakan |
| --- | --- | --- |
| rosdep: Cannot locate definition for ament_python | Stale buildtool_depend pada pure Python package. | Hapus <buildtool_depend>ament_python</buildtool_depend> dari tyrant_teleop dan tyrant_sim; pertahankan <build_type>ament_python</build_type>. |
| PluginValidationError launch_testing saat colcon test | pytest PlatformIO mengambil alih Python ROS. | Pastikan /usr/bin/python3 dan /usr/bin/pytest; jangan prepend .platformio/penv/bin; expose hanya symlink pio. |
| which pytest kosong | python3-pytest belum terpasang. | sudo apt install python3-pytest ros-humble-launch-testing ros-humble-launch-testing-ament-cmake. |
| ROS message/package tidak ditemukan | Workspace belum dibuild/disource. | source /opt/ros/humble/setup.bash; build; source install/setup.bash pada setiap terminal baru. |
| pio: command not found | PIO private env ada tetapi executable tidak exposed. | Symlink ~/.platformio/penv/bin/pio ke ~/.local/bin/pio; pastikan ~/.local/bin pada PATH. |
| Teensy upload permission denied / port tidak terlihat | udev/dialout/session belum refresh. | Install PlatformIO udev rules, add dialout, logout/login; cek pio device list, lsusb, /dev/serial/by-id. |
| micro-ROS node tidak muncul | Agent tidak running, serial device salah, firmware bukan micro-ROS production/mock/HIL, USB permission. | Start agent -v6; cek device; cek firmware profile; cek serial; lihat agent log. |
| Agent restart tetapi Teensy tidak reconnect | Connection lifecycle issue/device path berubah. | Cek /tyrant/communication/diagnostics; pastikan device stable; jalankan 4-state reconnect test; jangan reset sebagai workaround permanen. |
| MANUAL langsung SAFE | Host heartbeat >500 ms, transport disconnect, atau teleop mati. | Pantau /tyrant/system/health; publish heartbeat 10 Hz; cek agent/teleop. |
| ManualCommand `source_sequence` non-zero dan `command_fresh=true`, tetapi wrench `enabled=false`/zero | Communication/manual path sudah sehat; controller safety gate menolak state/config/dt. Paling umum pada bench tanpa sensor: `attitude_valid=false`. | Cek `/tyrant/simple_state`. Pada production pasang/validasi HWT905. Untuk software-only control gunakan `teensy41-hil` + `tyrant_plant_sim`; jangan bypass validity gate. |
| Thruster command semua zero pada teensy41 | Expected: production geometry belum tersedia/backend physical disabled. | Untuk software validation gunakan teensy41-mock-thrusters; jangan isi geometry generik. |
| AUTO ditolak AUTONOMY_NOT_READY | Expected jika mission/state/allocator/backend belum ready. | Cek /tyrant/system/health, mission status, build profile, state validity. |
| HIL state menjadi invalid | /tyrant/sim/state_input berhenti >200 ms. | Pastikan tyrant_plant_sim running dan topic rate stabil. |
| IMU READY tidak tercapai / stale | UART/baud/wiring salah atau no coherent samples. | Production binding HWT905=Serial1 @9600; jalankan smoke; cek stats/error_count. |
| Pressure invalid/stale | I2C/wiring/read failure atau no sample. | FullDepth=Wire @100 kHz; smoke test; cek pressure topic, read_attempts/read_successes. |
| Depth tidak nol di permukaan | Surface reference belum diset atau density/config beda. | Set surface reference saat sensor di posisi reference; cek status; gunakan density sesuai media test. |
| Controller axis salah arah | Linux controller mapping berbeda. | Gunakan tyrant_joy_probe, ubah index/invert/trigger settings YAML. |
| Compile error `rcl_context_t*` → `rmw_context_t*` pada destroy timeout | API micro-ROS meminta RMW context, code mengirim RCL context. | Gunakan `rcl_context_get_rmw_context(&support.context)` lalu pass hasil `rmw_context_t*`; jangan cast paksa. |
| Agent `create_session` lalu cepat `delete_object` + `session closed` | `createEntities()` gagal; sering karena default max publisher/subscription terlalu kecil. | Naikkan custom meta resource (development baseline 20 publishers/12 subscriptions), jalankan `clean_microros`, rebuild/upload, lalu cek entity creation lagi. |
| ManualCommand command_fresh=false atau source_sequence belum sesuai payload yang dikirim | Command valid belum diterima, umur penerimaan >250 ms, atau gate mode/communication membersihkan command. Sequence statis atau nol sendiri tidak berarti stale. | Cek ros2 topic info -v, publisher/subscriber, mode MANUAL dan heartbeat. Bandingkan payload dengan source_sequence. Uji timeout dengan menghentikan publisher; sequence meningkat membantu pelacakan paket. |
| IMU `bytes_received=0`, `valid=false` | HWT905 tidak dipasang/tidak powered, atau wiring/baud/UART salah. Jika sensor memang belum dipasang, kondisi ini expected. | Untuk hardware gunakan `teensy41-hwt905-smoke`, cek Serial1 @9600 dan wiring. Untuk software-only control gunakan HIL. |
| `ModuleNotFoundError: numpy` saat import `tyrant_interfaces` | ROS script dijalankan dengan Python PlatformIO atau `python3-numpy` belum tersedia pada Python sistem. | `which python3` harus `/usr/bin/python3`; keluarkan `.platformio/penv/bin` dari PATH; `sudo apt install python3-numpy`. |
| /joy aktif tetapi /tyrant/manual/command tidak ada | tyrant_teleop belum running, deadman tidak sesuai mapping, atau params file salah. | ros2 node list; jalankan tyrant_teleop dengan ps_controller.yaml; cek deadman/index menggunakan probe. |
| tyrant_teleop gagal: Error opening YAML file | Masih memakai placeholder NAMA_FILE.yaml atau path salah. | Gunakan ~/tyrant-simple/ros2_ws/src/tyrant_teleop/config/ps_controller.yaml. |
| Bluetooth controller terbaca tetapi mapping berbeda dari USB/controller lain | Axis/button numbering berasal dari Linux driver/device mapping. | Jalankan tyrant_joy_probe; satu input sekali; update ps_controller.yaml; jangan hard-code asumsi label PS. |

# 29. Code map — firmware

| Subsystem | File utama | Peran |
| --- | --- | --- |
| Main orchestration | src/main.cpp | Init, scheduler, ROS command handling, mode/safety, estimator/control/allocator/backend telemetry. |
| Connection | communication/connection_manager.cpp | 4-state ping/create/spin/destroy/reconnect. |
| micro-ROS interface | communication/ros_interface.cpp | Node, publishers, subscribers, executor callbacks, message serialization. |
| Safety | safety/safety_manager.cpp | Heartbeat/transport health, mode gate, propulsion/autonomy authority. |
| Vehicle mode | vehicle/vehicle_mode.cpp | Mode transition policy. |
| IMU driver | sensors/drivers/hwt905_driver.cpp | WitMotion raw frames → mounted IMU sample. |
| Depth driver | sensors/drivers/fulldepth_pressure_source.cpp | MS5837 pressure/temp source. |
| Sensor manager | sensors/sensor_manager.cpp | Source lifecycle, sample/health/freshness/stats, depth conversion path. |
| Simple estimator | estimation/simple_state_estimator.cpp | Complementary roll/pitch + 2-state yaw KF + depth validity. |
| Manual controller | control/stabilized_manual_controller.cpp | Surge/sway effort + depth/attitude/heading stabilization. |
| TAM | control/thruster_allocation.cpp | Build 6x8 matrix from [d_i; r_i × d_i]. |
| Allocator | control/thruster_allocator.cpp | Damped least-squares normalized allocation. |
| Backend | actuation/thruster_backend.cpp | Select disabled/mock/PWM behavior + watchdog/status. |
| PWM mapping | actuation/thruster_output_mapper.cpp | Normalized actuator command → pulse mapping/deadband/inversion. |
| Mission | application/mission_manager.cpp | Time/depth/heading step sequencing. |
| Emergency | application/emergency_recovery_manager.cpp | Zero-thrust/controlled-surface recovery policy. |
| SysID | sysid/sysid_recorder.cpp | Sessioned synchronized dataset at bounded rate. |
| HIL input | simulation/sim_state_source.cpp | Accept/validate/freshness of ROS-injected SimpleState. |
| Scheduler | scheduler/scheduler.cpp | Control/safety/health/heartbeat cadences. |

# 30. Code map — ROS packages

| File | Fungsi |
| --- | --- |
| tyrant_teleop/teleop_node.py | Controller event handling, deadman, joy timeout, heartbeat, ManualCommand/mode/depth-ref publication. |
| tyrant_teleop/mapper.py | Pure mapping/deadzone/trigger normalization; unit-testable tanpa ROS. |
| tyrant_teleop/joy_probe.py | Axis/button calibration helper. |
| tyrant_teleop/config/ps_controller.yaml | Per-device mapping dan safety timing teleop. |
| tyrant_sim/plant_node.py | Reduced plant HIL node. |
| tyrant_sim/model.py | Synthetic reduced dynamics. |
| tyrant_sim/profiles.py | Step/PRBS/chirp deterministic excitation. |
| tyrant_sim/excitation_node.py | Safe sequencing MANUAL + SysID experiment. |
| tyrant_sim/config/reduced_plant.yaml | Synthetic HIL coefficients; development only. |

# 31. Recommended terminal layout untuk local test

| Terminal | Isi |
| --- | --- |
| T1 | micro-ROS agent (source ROS + old agent workspace). |
| T2 | Project ROS environment + ros2 topic/node monitoring. |
| T3 | Teleop/joy atau HIL plant. |
| T4 | Publisher manual/heartbeat/mission/emergency/SysID sesuai test. |
| T5 optional | rosbag/PlotJuggler/log inspection. |

Setiap terminal ROS baru minimal perlu:

source /opt/ros/humble/setup.bash

source ~/tyrant-simple/ros2_ws/install/setup.bash

Terminal agent additionally source workspace agent lama, bukan workspace project sebagai pengganti agent.

# 32. Daily startup checklist

`cd ~/tyrant-simple` dan pastikan branch/source yang benar.

`source /opt/ros/humble/setup.bash` + `source ros2_ws/install/setup.bash`.

Cek `which python3`, `which pytest`, `which pio` bila environment berubah.

Connect Teensy dan cek device stable; prioritaskan `/dev/serial/by-id/...` untuk agent jika tersedia.

Start micro-ROS agent.

Cek `/tyrant/communication/diagnostics`, `/tyrant/system/health`, `/tyrant/simple_state`.

Jika memakai hardware sensor, cek `/tyrant/sensors/imu` dan `/tyrant/sensors/pressure`.

Baru start teleop/HIL/SysID/mission sesuai kebutuhan.

Pastikan SAFE/zero command sebelum menutup sesi.

# 33. Gate sebelum pindah workflow ke Jetson

| Gate | Status yang harus dicapai |
| --- | --- |
| ROS build | tyrant_interfaces, tyrant_teleop, tyrant_sim build bersih. |
| ROS tests | Seluruh Python tests pada revisi aktif: 0 failures/errors. Catat jumlah aktual; baseline v1.2 berisi 18 test. |
| Firmware compile | Production, mock, HIL, PWM skeleton, sensor smoke profiles compile. |
| Embedded tests | Core + connection tests PASS pada Teensy. |
| micro-ROS | Node/topic lengkap, pub/sub dua arah PASS, manual/heartbeat timeout PASS, kedua boot-order PASS, agent restart reconnect 10x tanpa reset Teensy, long-run 30 menit tanpa session collapse. |
| Sensors dan depth | HWT905 valid/stable di bench. FullDepth fisik boleh DEFERRED; depth-dependent software harus lulus HIL. |
| Estimator | Attitude bench stationary 5–10 menit bounded; depth dan closed-loop behavior divalidasi melalui HIL saat FullDepth ditunda. |
| Teleop | Deadman/manual/SAFE/timeout behavior correct. |
| Mock/HIL | End-to-end command → mock thruster → plant → state loop works. |
| SysID | Session START/STOP/sample pipeline works; HIL data explicitly treated synthetic. |

Gate ini menilai kesiapan memindahkan workflow software ke Jetson. Selama FullDepth fisik ditunda, validasi HWT905 tetap dilakukan di bench dan skenario yang membutuhkan depth diuji melalui HIL. Catat sumber state dan evidence untuk setiap gate.

Setelah gate software/bench ini terpenuhi, setup Jetson mereplikasi known-good environment dan deployment workflow. Pemasangan, smoke test, surface reference, dan kualifikasi FullDepth nyata tetap wajib sebelum kontrol depth fisik. HIL tidak menggantikan kualifikasi sensor atau kendaraan nyata.

# 34. Quick recovery commands

# Reset ROS package build artifacts (selective)

cd ~/tyrant-simple/ros2_ws

rm -rf build/tyrant_sim build/tyrant_teleop install/tyrant_sim install/tyrant_teleop

# Rebuild all project packages

source /opt/ros/humble/setup.bash

colcon build --symlink-install

source install/setup.bash

# Check Python isolation

which python3; which pytest; which pio

# Check Teensy device

pio device list

ls -l /dev/serial/by-id/ 2>/dev/null || true

# Compile production without upload

cd ~/tyrant-simple/firmware/teensy

pio run -e teensy41

# Upload safe production

pio run -e teensy41 -t upload

# 35. Catatan validasi dan batas dokumen

Angka, topic, node, config, dan profile pada dokumen ini diambil dari snapshot tyrant-simple terbaru yang memuat HIL + SysID + PWM skeleton.

Host-side generation validation sebelumnya melaporkan 155 Teensy test cases, 8 teleop Python tests, dan 10 simulator/profile tests; actual laptop/Teensy execution tetap harus dicatat sebagai local evidence.

Sinkronisasi v1.3 mempertahankan hasil historis dan tidak menaikkan status PENDING/DEFERRED menjadi PASS. Bukti pengujian baru harus mencantumkan revisi kode/config, profile firmware, perangkat, serta hasil aktual.

Geometri sintetis, HIL dynamics, PWM placeholder 1100/1500/1900 us, dan control gains saat ini adalah development configuration, bukan final vehicle calibration.

Production AUTO sengaja readiness-gated. Jangan “memperbaiki” AUTONOMY_NOT_READY dengan mem-bypass safety gate.

micro-ROS agent lama boleh direuse selama Humble-compatible; ConnectionManager firmware yang bertanggung jawab melakukan reconnect client lifecycle.